How to add extended PIDs in Torque Pro App

Need copy file *.csv to /sdcard/.torque/extendedpids/

The structure of the CSV file will be:

Name,ShortName,ModeAndPID,Equation,Min Value,Max Value,Units,Header

Coolant Temperature,ЕСТ,2102,A/255*192,-49,-50,140,C

In Torque app choose Settings->Manage extra PIDs/Sensors->Add predefined set and select your file *.csv. Video

Then add new displays in Realtime information window.

Manual adding PID Edit

Go to Settings->Manage Extra PIDs->Add custom PID

Fill fields in window "OBD2 PID Editor":

OBD2 Mode and PID. Max 3 bytes in hex or 6 hexadecimal digits

Example: 210CDF

Long Name, that showed in list of PID

Short name. Shown in display name in dashboard

Minimum Value. The minimum value that showed in result even the result is smaller than min value??

Maximum Value. The maximum value that showed in result even the result is bigger than max value??

Scale Factor.

Unit Type

Equation. Formula for calculation result value.

Index number of byte in response denoted by Latin alphabet:

A-1 byte, B-2 byte,.., Z-26 byte,AA-27 byte,AB-28 byte,...BA,BB,...

Instead of using byte from response, You can use value of internal pid in square brackets, like [ff1201]

Supported arithmetical operations:

A+B sum

A-B subtraction

A*B multiplication

A/B dividing

A<8 shift to the left. Equal A*256

signed(A). A means as signed value - assuming highest bit is a sign bit

ABS(x) - absolute value

{A:4} Get bit from byte

A&256 bitwise comparison

VAL(“pid full name”) - reference to other PID added with plugin in the equation

P.S. Decimal separator character is "."(dot) Example: 2.56