2025-05-18 10:27:09 PM

# Jaguar F-Type Calipers and Rotors upgrade

## Front wheel caliper removal procedures

- Remove front wheel

- Remove brake pad sensor

- Remove the anti-rattle spring in the centre of the spring until either side is released

- Unscrew Caliper (Take off dust cover and unscrew 2 x 9mm hex) from support bracket

- Remove and discard pads

- Remove brake line from caliper and clamp down brake line

- Remove Caliper support bracket (#2 below diagram) – 2 x 15mm hex bolts

- Remove rotor

- Make sure mating surfaces (i.e. Bracket mounting area and wheel hub) are clean

## Front wheel caliper installation procedures

- Install new Caliper support bracket using new bolts (Torque: 115Nm)

- Clean and Install rotor

- Install pads into new Caliper and reinstall caliper (Torque: 58Nm)

- Hook up brake lines (Torque: 42Nm)

- Replace brake pad wear sensor

- Brake system bleeding

## Rear wheel caliper removal procedures

- Perform Electric Park brake service mode activation

- Remove Rear wheel

- Remove brake pad sensor

- Disconnect parking brake actuation connector

- Unscrew Caliper from support bracket unscrew 2 x 13mm hex

- Remove and discard pads

- Remove brake line from caliper and clamp down brake line

- Remove Electric Park brake actuator from caliper

- Remove Caliper support bracket 2 x 15mm hex bolts

- Remove rotor

- Remove Splash shield

- Install Electric Park brake actuator into new caliper (New O-ring seal and bolt kit) (Torque: 42Nm)

## Rear wheel caliper installation procedures

- Install new Splash shield

- Install new rotor

- Install Caliper support bracket using new bolts (Torque: 115Nm)

- Clean and Install rotor

- Install pads into new Caliper and reinstall caliper (Torque: 58Nm)

- Hook up brake lines (Torque: 42Nm)

- Replace brake pad wear sensor

- Brake system bleeding

## Additional Reference

Brake system – general information – Page 2428

Front Wheel Speed Sensor Page 5279

Rear Wheel Speed Sensor Page 5310

Brake system bleeding Page 28173