New XJ6 Bonnet Stay Installation Instructions

6-15-2019

- Remove OEM Bonnet Stay Assembly

- You will need to improvise a temporary hold open device for hood

(broom handle, Length of PVC pipe etc.). This assumes a no wind location; if you are outside, additional precautions are advised)

- Remove two forward  bolts (attaching OEM stay assembly to bonnet)

- Remove two aft ¼” bolts (attaching angle fittings to radiator support frame)

- Clean, prime & paint area as desired

- Install New XJ6 Bonnet Stay

- Assemble New XJ6 Bonnet Stay per drawing

- Install two Angle Fittings to radiator support frame with two ¼” x ½” NF Bolts & Lock Washers (the longer leg of the angle fitting should be against radiator frame)

- Temporarily Install  ¼ x 1” pivot bolt into two angle fittings for alignment  and tighten the ¼” x ½” mount Bolts (there may be small gap between angles)

- Install aft Link with one ¼” x 1” Bolt, two Washers and a Lock Nut on to Angle Fittings

- Tighten Lock Nut to firm, then back off 1 Nut flat

- Install forward link to Bonnet with 5/16” Bolt, two Washers and a Lock Nut

- Tighten Lock Nut to firm, then back off 1 Nut flat

- Adjustment of Lock Knob

- Install Lock Knob on ¼”-20 x 1-1/4” bolt, tighten firm (links straight)

- Back off Lock Knob 1/4 of a turn

- Push on #10 Thread Protector on to ¼”Bolt inside Knob as far as possible (ensures knob does not vibrate off)

- All links should move freely, and bonnet should close without binding

- If any binding is found, loosen Lock Nut one flat at a time to free mechanism

- Some have reported that a notch in the plastic cover on top of bumper is required to accommodate  the hinge arm at 50” open position. (~ 5/16” x ¼” notch)