[image: image1.png]



PRESS INFORMATION

FOR RELEASE: Embargoed until 00.01 hrs GMT, 28th December 2000 

CONTENTS

1
OVERVIEW


NEW JAGUAR S-TYPE RANGE SPEARHEADED                  

BY HIGH PERFORMANCE S-TYPE R


1-12


KEY TECHNOLOGIES AT A GLANCE


13-17

2
NEW S-TYPE RANGE IN DETAIL


POWERTRAIN





18-24


VEHICLE DYNAMICS




25-30


NEW INTERIOR DESIGN & ENGINEERING


31-32


TELEMATICS





33-35


SAFETY TECHNOLOGY




36


EXTERIOR DESIGN & ENGINEERING


37-38


MANUFACTURING




39-41

3
STANDARD AND OPTIONAL EQUIPMENT


42-45

4
TECHNICAL SPECIFICATIONS



046-47


DIMENSIONS





47

5
PRICE LIST





48

NEW JAGUAR S-TYPE RANGE SPEARHEADED

BY HIGH PERFORMANCE S-TYPE R

An all-new supercharged 4.2 litre S-TYPE R — the most powerful Jaguar saloon ever - and a new 2.5 litre V6 entry model spearhead the launch of an extended and refined  S-TYPE range, now eight models strong. All variants feature a completely redesigned interior, which blends traditional luxury and craftsmanship with advanced technology, including optional touch-screen telematics displays and class-leading occupant safety systems.

A new six-speed ZF automatic transmission — an industry first in the executive segment — is available across the new S-TYPE range, complementing the refined power delivery of the new Jaguar 2.5 AJ-V6 engine developing a class-leading 201 bhp (DIN), and larger capacity 4.2 AJ-V8. The new refined 4.2 litre is available in both 300 bhp (normally aspirated) and 400 bhp (supercharged) forms. The all-new front suspension and extensively revised rear suspension deliver outstanding ride comfort with poised, agile handling, in keeping with the Jaguar tradition.

Building on success

S-TYPE's launch in 1999 marked the beginning of a major product-led transformation for Jaguar. Following the launch, the company's worldwide sales virtually doubled and S-TYPE sales to date have exceeded 135,000 cars.

"Building on the success of the S-TYPE, we have refined and developed the range to appeal to an even wider market and to strengthen our position as one of the leading players in the premium saloon segment. Reaffirming Jaguar's core marque values of distinctive style, refined power, agile handling, supple ride and luxurious comfort, the new S-TYPE is even better equipped and more accessible than ever, with a choice of engines and specifications to suit individual tastes and budgets"

MIKE BEASLEY, MANAGING DIRECTOR

Exhilarating new S-TYPE R – high performance flagship

Leading the radically revised S-TYPE line-up is Jaguar’s most powerful saloon yet, 

the formidable S-TYPE R. Its hard-charging abilities are delivered with the full complement of Jaguar luxury, comfort and style, for a competitive £47,400 on-the-road.

With 400 horsepower on tap, Jaguar’s new 4.2 litre supercharged V8 engine propels the S-TYPE R from zero to 60 mph in just 5.3 seconds and then on to an electronically limited top speed of 155 mph. What’s more, the engine’s broad torque band and the new, six-speed automatic ZF transmission ensure equally rewarding response and flexibility in everyday driving conditions.

"The S-TYPE R has a dual personality, in line with Jaguar’s philosophy for our ultra high performance flagship vehicles. The new car is inherently quiet, refined and comfortable – but when stirred, it becomes a true performance machine, satisfying the demands of the driving enthusiast. Power is instantly available, without ever being intimidating or overly demanding of the driver."

MIKE CROSS, CHIEF ENGINEER, VEHICLE INTEGRITY

To match the powerful engine and advanced transmission, the S-TYPE R is equipped with sports suspension, race-engineered Brembo brakes and the latest generation of Jaguar’s acclaimed Computer Active Technology Suspension (CATS) to optimise ride and handling. Now enhanced for improved stability and comfort, particularly during cornering and braking manoeuvres, CATS uses uprated springs for increased roll-stiffness and electronically controlled, two-stage adaptive dampers for ride refinement. CATS now automatically switches the rear dampers slightly ahead of the front pair at low speed to reduce transient understeer for improved agility and turn-in performance. At high speed, the system automatically switches the front dampers ahead of the rears to increase transient understeer for improved stability. 

The car’s luxurious, sports-style interior features leather-trimmed R Performance steering wheel and sports seats, which are 16-way adjustable, with memory on the driver’s seat. The new-look S-TYPE facia and door casings are finished in distinctive, grey bird’s-eye maple veneers.

Exterior design cues clearly signal the spirited character of the S-TYPE R.  

They include a rear spoiler for high speed stability, exclusive (18 x 8 inch) ‘Zeus’ 

alloy wheels (18 x 9.5 inch on the rear), tinted lamps all round (including xenon headlights) and a bright-finish mesh front grille with body-coloured surround – the hallmark of a supercharged Jaguar.  

Reflecting its ultra-high performance pedigree, the S-TYPE R carries the same distinctive red R logo that will feature on the nose cone of the new Jaguar R3 Formula One racing car for the 2002 season. First seen on the R Coupe Concept at the 2001 Frankfurt autoshow, the R badge will feature on all future Jaguar R Performance derivatives and options. 

S-TYPE 2.5 V6: new entry point and outstanding value

At the other end of the range, the new 2.5 litre V6 model, priced at only £24,950 

on-the-road, combines a feature-rich standard specification with highly competitive ownership costs and a class-leading power output of 201 bhp (DIN). The S-TYPE 2.5 V6 provides unrivalled value-for-money in the executive segment, with a price advantage of £450 over the equivalent BMW 525i and an £840 advantage over the Mercedes E240 model (price comparison correct as of 27th November 2001). In addition, the new 

S-TYPE 2.5 V6 offers a richer standard feature specification than its German competitors, including electrically adjustable seats (6 way driver, 4 passenger way), electrically adjustable steering column and 60/40 split folding rear seats.

The standard specification of the 2.5 V6 entry-level model includes a wide range of features and advanced technologies, which provide real customer benefit in terms of comfort, convenience and safety:

· Bronze-coloured bird’s-eye maple veneers and leather gear knob.

· Automatic dual-zone climate control, (combined odour and pollen filter)

· Electrically adjustable seats (6 way driver, 4 way passenger) & steering column

· 60/40 split-folding rear seats

· Electronic Parking Brake

· Automatic headlamp activation

· Dynamic Stability Control, with traction control and anti-lock brakes, 

Including new panic-assist feature

· 140 watt Alpine sound system with four speakers

· Jaguar’s innovative Adaptive Restraint Technology System (A.R.T.S.), with 

ultrasonic occupant sensing

· Side curtain airbags and energy-absorbent front seat backs

· 16 inch 'Helios' alloy wheels with an optional 17 x 7.5 inch 'Sport ' alloy wheel

S-TYPE 2.5 SE, 3.0 SE and 4.2 V8: contemporary Jaguar luxury and refinement

Moving up the range, the new 2.5 litre engine is also available in luxurious SE (Special Equipment) form, as are the 3.0 litre V6 and the new, larger capacity, naturally aspirated 4.2 litre V8 for those who demand the ultimate in Jaguar refined power.

The established 3.0 litre AJ-V6 engine delivers a class leading 240 bhp (DIN) and in common with its new 2.5 litre stablemate features improved three-stage variable geometry induction and continuously Variable Cam Phasing systems for faster engine response and enhanced refinement.

The new 300 bhp (DIN) 4.2 litre AJ-V8 engine develops seven per cent more power 

and eight percent more torque than the previous 4.0 litre power unit. This benefits driveability, refinement and performance throughout the rev range. At the same time fuel economy and emissions are also improved.

The emphasis of the SE variants is on contemporary luxury in true Jaguar style.  Standard features of the SE are:

· New, six-speed ZF automatic transmission

· Leather seats with electric adjustment (six-way front passenger and eight-way 

driver, with memory)

· Wood and leather steering wheel with integral controls for in-car entertainment 

and standard-fit cruise control

· Metallic paint and front fog lamps

· Electrochromatic rear view and door mirrors

· ‘Spirit’ design alloy wheels (16 x 7.5 inch) with optional (16 x 7.5 inch)  

· 'Artemis' alloy wheel on 2.5 and 3.0 litre models

· ‘Kronos’ design alloy wheels (17 x 7.5 inch) on the 4.2 V8

On-the-road price are £28,900 for the 2.5 SE, £31,400 for the 3.0 SE and £36,000 for the 4.2 V8.

S-TYPE 2.5 and 3.0 Sport: distinctive, purposeful, dynamic 

The new S-TYPE Sport models are designed for predominantly younger buyers who demand a more purposeful, dynamic character, both inside and out. With their distinctive colour-coded exterior, leather sports interior, up-rated sports suspension and brakes, the new Sport models are cars for the true driving enthusiast. The S-TYPE Sport is available with a choice of two powerful V6 engines: the 2.5 litre V6 (201 bhp) and the 3.0 litre V6 (240 bhp). On-the-road prices start at £27,450 for the 2.5 litre model, and £29,950 for the 3.0 litre V6 Sport. In summary, the distinguishing features of the Sport models are:

· Leather sports seats with eight-way power adjustment for driver (six-way for 

front passenger)

· Leather sports steering wheel

· Grey-stained bird’s-eye maple veneers

· ‘De-chromed’ exterior details: body-coloured front grille surround / boot plinth/ 

bumper blades and black window surrounds

· Sports suspension with uprated springs and dampers

· 5-speed Getrag Manual gearbox

· Six-speed automatic ZF transmission is a cost option

· Larger ventilated front brake discs (320mm x 30mm)

· Unique-design 17 x 7.5 inch ‘Herakles’ alloy wheels 

· Two optional R Performance alloy wheel designs are also available – 'Indianapolis' 

     
(18 x 8 inch) and 'Monaco' (18 x 8 inch) which may also be specified on any new 

     
S-TYPE model

"The emphasis of the engineering programme for 2002 has been very much on refinement and extending the appeal of the S-TYPE range. The new 2.5 V6, enlarged 4.2 V8 and new six-speed automatic deliver effortless, refined Jaguar power, while the supercharged R unequivocally establishes S-TYPE's performance credentials. For the interior, we listened carefully to our customers to create a new cabin environment, which combines craftsmanship with leading-edge technology.  Beneath the surface, S-TYPE's renowned ride and handling qualities are further enhanced through numerous refinements, including a new front suspension.S-TYPE now offers more luxury, performance and driving pleasure than ever." 

PHIL HODGKINSON, CHIEF PROGRAMME ENGINEER

State-of-the-art ZF six-speed automatic transmission –  an industry first in the executive segment

The Jaguar S-TYPE is the first car in its class to offer a state-of-the-art ZF 6HP26 six-speed automatic transmission, delivering refined, accessible performance with seamless gearchanges. It is standard with both supercharged and naturally aspirated V8 engines and V6 SE models. It is also available as an option across the new S-TYPE line-up, including the AJ-V6 2.5 litre entry model.

Jaguar's unique ‘J-gate’ gear selector system is uprated to suit the new six-speed transmission and now features electronic control, contributing to smoother shift operation and ergonomics.

Electronic Parking Brake with fully automatic operation - 

another first for Jaguar (on all automatic transmissions)
All models are fitted with an Electronic Parking Brake, which replaces the conventional handbrake lever. It is operated by a console-mounted switch, providing 

both ease of use and space saving benefits. With the automatic transmission the brake is also applied automatically whenever the ignition key is removed, and released whenever the gear selector is moved from the ‘park’ position.

The braking system on the new S-TYPE features new large diameter, ventilated front and solid rear discs, a four channel anti-lock system, now with panic assist braking, together with Electronic Traction Control and Dynamic Stability Control system as standard. New, lightweight steel-braided brake hoses deliver improved durability.

Electronically adjustable pedals with memory for the optimum driving position
One of the new S-TYPE’s most innovative features is its optional, electronically adjustable pedal system. Used in conjunction with the seat, steering wheel and  exterior mirror adjustment, it helps achieve the optimum driving position.  Furthermore, pedal location can be programmed with two memorised positions 

for different drivers.

All-new front and revised rear suspensions plus stiffer body structure deliver enhanced chassis dynamics

All models are fitted with all-new front and revised rear suspensions, enhancing

S-TYPE’s outstanding ride comfort and poised, agile handling.  

The new front suspension incorporates forged aluminum control arms, new stiffened cast aluminum front knuckles and new front subframe. The rear suspension is extensively revised and includes a new rear subframe, more rigidly mounted to the body, new control arms and knuckles, and a new toe-link for improved rear wheel alignment.

Jaguar engineers have developed the suspension system to accommodate the stringent ride and handling standards set for the high performance supercharged S-TYPE R. Many of these developments are included in the standard suspension design, benefitting all models in the range.

The new S-TYPE's highly stable body structure, now 10 per cent stiffer, delivers significant benefits. It ensures highly consistent door-to-body shut lines and door seal location, benefiting both cosmetic appearance and vehicle refinement. S-TYPE's stiff body also contributes to a smooth, stable ride by providing a rigid mounting for the new suspension components, allowing them to work to optimum efficiency. This helps to isolate the effects of road irregularities, minimising the transmission of noise and vibration into the cabin. It also contributes to an overall feel of robustness in all driver interfaces, including steering, pedals and instruments.

Overall vehicle weight is reduced by an average 50 kg through increased use of lightweight materials including forged aluminum control arms, the use of magnesium for front seat frames, facia structure and cross-car beam, and the use of lightweight steel in the rear seat structure.

Subtle exterior styling refinements – building on success

"S-TYPE’s styling maintains Jaguar’s long tradition of producing some of the most beautiful, innovative and distinctive saloon cars in the world.  With the new range, we took the opportunity to incorporate subtle exterior styling enhancements, building on a design that has proved extremely successful with our customers."

IAN CALLUM, DIRECTOR OF DESIGN

A revised grille with an integrated Jaguar ‘growler’ emblem is the main visual change to the front of the car,while S-TYPE’s distinctive profile remains unchanged. New window surrounds chrome-finished on entry and SE models, new exterior mirrors (with optional electrochromatic function), a range of new alloy wheel designs and new badging contribute to the subtle exterior detail refinements.

Redesigned interior, a blend of Jaguar craftsmanship and advanced technology.

"The interior of the new S-TYPE harmoniously blends the latest technology and safety engineering with hand-crafted natural forms and surface materials. The result is a luxurious, functional and contemporary environment, with a distinctive Jaguar character."

IAN CALLUM, DIRECTOR OF DESIGN

The extensively revised S-TYPE interior delivers even greater comfort and superior levels of luxury. It features a new facia with a full-width bird’s eye maple veneer panel and a redesigned centre console that houses the optional 7-inch wide-screen LCD touch-screen display. New instruments with ‘racing green’ dials and slender pointers reinforce the car’s sporting feel, craftsmanship and quality.

More comfortable, all-new seats suit the specific characteristics of each of the 

models in the range, and driver comfort can be enhanced further with the optional, electronically adjustable foot pedals. Detail improvements include new steering 

column switchgear, one-touch activation with anti-trap for the electric windows 

and an optional glass sunroof. 

Leading-edge safety systems, including Adaptive Restraint Technology System 

The S-TYPE’s comprehensive occupant safety systems now include Jaguar's leading-edge Adaptive Restraint Technology System (A.R.T.S.), first seen on the XK Series sports cars, as standard fitment. The Adaptive Restraint Technology System, uses ultrasonic sensing technology to monitor the position of the front passenger, while an array of other sensors detect the front passenger’s weight, the position of the driver in relation to 

the steering wheel, the usage of seatbelts and, should a crash occur, the severity of 

the impact. The system greatly reduces the risk of airbag-related injuries caused by inappropriate airbag deployments.

Further safety enhancements include front and rear side-curtain airbags, energy-absorbent front seat backrests, incorporating an anti-whiplash mechanism, and front and rear seat belt pre-tensioners. In addition, xenon headlights are introduced across the range, being standard on the new S-TYPE R and a cost option on other models.

The already extensive use of advanced steels in critical areas of the body is increased for the new S-TYPE, with the addition of ultra-high strength steels in key areas such as sills, pillars, and cantrails for enhanced structural integrity and to accommodate side-curtain airbags.

Latest multimedia systems

Leading-edge multimedia systems, pioneered on the X-TYPE, are now available 

on S-TYPE. The new optional LCD touch-screen display, employing thin film 

transistor technology (TFT), provides finger-tip control of climate, audio and navigation systems, and (where fitted) television tuner and dual-band, fixed GSM telephone.  

This exceptionally high level of integration is facilitated by fibre optics to provide 

fast, high-quality data transfer.

The JaguarVoice voice-activation system, a world-first for Jaguar when launched on S-TYPE in 1999, provides voice control for primary functions of the audio, telephone and climate control systems, as well as - for the first time - the satellite navigation system and TV tuner, where fitted. Maintaining Jaguar’s lead in the field of voice activation, this system is now available in French, German, Italian, Spanish and 

English, including North American and Australian accents.

DVD sat nav with postcode destination programming

A multi-function, satellite navigation system is available as a cost option. 

The system employs DVD technology to deliver highly accurate information, 

virtually instantaneously. The navigation system has 'voice guidance', selectable 

in nine languages and mapping covers 17 major markets. It features postcode entry destination programming - simply 'input' the postcode of the required destination and the satellite navigation automatically programmes the optimum route.

Premium sound system option

Each model in the new S-TYPE range is specified as standard with a 140 Watt Alpine sound system with four full-range, door mounted speakers and a keypad for both channel selection and telephone dialling, where the optional in-car phone is fitted.

The optional 320 Watt, ten-speaker Alpine Premium Sound System features Digital Signal Processing (DSP) with three occupancy settings, optimising sound quality and stereo effects for either the driver, front seat passenger or all occupants.

S-TYPE a rational choice for the corporate sector

With the addition of the 2.5 litre V6 model, S-TYPE now offers an expanded range 

of models and derivatives to satisfy the needs of the business user, moving Jaguar 

into the heart of the corporate sales sector.

While the Jaguar marque is already a well-established choice at boardroom level, 

the S-TYPE – and the 2.5 litre V6 model in particular – will make even more Jaguars available to many more mainstream company car drivers, especially 'user-choosers' who are able to select from a list of company car options.

Whole-life cost suppliers forecast excellent residual values for the S-TYPE range, in line with the previous model and the X-TYPE. Good residuals mean that Jaguar can offer highly competitive lease rates. For both corporate and private purchasers, Jaguar Financial Services can offer attractive tailor-made deals, with insurance and vehicle servicing built into the monthly payments so that customers know exactly what they will pay, with no hidden extras. 

While running costs and residual values dictate fleet choice lists, company car 

drivers tend to select vehicles on image, desirability, exclusivity, ride, handling and performance. S-TYPE satisfies all these requirements. In addition, with the introduction of a smaller engine, highly competitive pricing and lower CO2 emissions, S-TYPE presents significant tax advantages for UK company car drivers. The new 2.5 V6 in particular will also be an attractive proposition for the increasing number of British employees who are choosing to opt out of their company car schemes due to changes to benefit-in-kind (BIK) taxation.  

Competitive cost of ownership

Achieving competitive ownership costs throughout its life was a prime consideration 

for the new S-TYPE range. S-TYPE's keen prices and rich standard feature specifications provide a significant advantage over equivalent BMW 5 Series and Mercedes E-Class models in terms of both list price and value-for-money. For example, the S-TYPE 2.5 V6 entry model at £24,950 on-the-road, enjoys a list price advantage of £450 over the equivalent BMW 525i and an advantage of £840 over the Mercedes E240 (price comparison correct as of 27th November 2001). The S-TYPE 2.5 V6 also offers a richer standard feature specification than its German competitors, including electrically adjustable seats (6 way driver, 4 passenger way), electrically adjustable steering column and 60/40 split folding rear seats. 

Higher up the range, the S-TYPE 4.2 V8, at £36,000 on-the-road, enjoys a list price advantage of £4,110 over the BMW 540i, while the S-TYPE R, the performance flagship, priced at £47,400 on-the-road, undercuts the price of the BMW M5 by £4,600. Unlike the M5, the S-TYPE R also features as standard, a state-of-the-art six speed, electronically controlled ZF automatic transmission.

Comprehensive security systems, fully integrated into the vehicle electronics, exceed the stringent British Insurance Industry's Criteria for Vehicle Security and include engine immobiliser, perimeter alarm, intrusion sensing, deadlocking, two-stage unlocking and automatic drive-away locking. The Category 1, Thatcham-approved security system, together with highly competitive repair costs, should result in competitive insurance groupings, benefiting premiums. 

The S-TYPE's comprehensive safety systems can also help to reduce ownership costs.  

An added benefit of the advanced occupancy sensing system is the avoidance of repair costs associated with unnecessary airbag deployments, particularly when the passenger seat is unoccupied and in low-impact collisions when the occupants are belted.

Jaguar's commitment to after-sales customer care ensures that the S-TYPE, like every new Jaguar, is backed by one of the most comprehensive warranties and service support in the industry:

· 3 year / 60,000 mile mechanical and electrical warranty (extendable up to 5 years / 

100,000 miles at extra cost)

· Total Incident Management, backed by the RAC, including 24 hour rescue, 

· Full Mondial European Emergency Assistance; 

· 3 year, unlimited mileage paint surface warranty

· 6 year, unlimited mileage corrosion (perforation) warranty.

In common with the rest of the Jaguar range, routine service intervals are at 

10,000 mile (16,000 km) or 12 monthly intervals.

KEY TECHNOLOGIES AT A GLANCE

The following technologies are standard specification, except where stated otherwise.  New or additional features are highlighted in italics and bold.

Powertrain

· New 2.5 litre AJ-V6 engine develops class leading 201 bhp (DIN)

· The updated 3.0 litre AJ-V6 at 240 bhp (DIN) also delivers class leading power 

with flatter torque curve and improved fuel economy (8% better on combined 

cycle with manual gearbox)

· State-of-the-art lightweight ZF 6 HP 26, six-speed automatic transmission (standard on SE models, 4.2 V8 and supercharged S-TYPE R) - a world first in the executive segment

· In 'Sport' mode, S-TYPE R models incorporate adaptive gearshift strategies for improved transmission responsiveness to driving style and road conditions

· Lightweight 2.5 litre and improved 3.0 litre, 24-valve, four-cam AJ-V6 engines feature continuously variable cam phasing and variable geometry air intake

· 4.2 litre, 32-valve, four-cam AJ-V8 engines – available in 300 bhp (DIN) naturally aspirated and, 400 bhp (DIN) supercharged  forms incorporating continuously variable cam phasing. Improved performance, refinement, economy and emissions

· Bespoke 32-bit powertrain electronic controller which integrates both engine 

and transmission functions, with 1 Megabyte memory capacity

Vehicle Dynamics

· All-new front suspension – with forged aluminium front upper / 

lower control arms, new stiffened cast aluminium front knuckle

· Retuned springs, dampers, bushes and anti-roll bars

· Revised rear suspension, including new rear subframe, more rigidly mounted to the body, with new control arms and knuckles, and a new toe-link

· Improved Computer Active Technology Suspension (CATS) – standard on R model - with more sophisticated damper control during cornering and braking for  -enhanced ride comfort and stability

· Traction control standard on all models

· Dynamic Stability Control (understeer/oversteer) now standard on all models

· Anti lock brakes with Panic Brake Assist, which detects a 'panic brake' application, 

ensuring maximum braking efficiency

· The front brake system of all models except the R incorporate twin–piston, aluminium sliding callipers.

-
30 mm width x 300 mm diameter ventilated discs on the V6 (except Sport)

· 30 mm x 320 mm diameter ventilated discs on the V8 and V6 Sport

· At the rear, all V6/V8 models have 20 mm x 288 mm discs.

· S-Type R features 32 mm x 365 mm ventilated front discs and 15 mm x 330 mm solid rear discs, with four–piston aluminium callipers from racing brake sepcialist, Brembo

· Steel-braided brake hoses, which are durable yet lightweight

· Electronic parking brake (auto transmission) - industry first in the executive segment

· Jaguar's ‘J-gate’ gear selector has been adapted to suit the new six-speed transmission, contributing to smoother shift operation and ergonomics with shift by wire 

(on manual side of the J-gate – gears 2, 3, 4 and 5)

· Variable ratio, speed proportional power steering

New Interior Design and Engineering

· New facia with a full-width bird’s eye maple veneer panel and lightweight 

magnesium understructure

· New seats introduced to provide increased comfort, with Classic, Sports and 

· R performance seat designs

· New instruments with ‘racing green’ dials and slender pointers

· New Electronic Parking Brake - operated by a compact, console-mounted switch. 

· With auto transmission the brake is also applied automatically on ignition key removal and, is released when the gear selector is moved out of 'park'

· New electrically adjustable foot pedals with memory (option), linking in with seat, 

steering wheel and exterior mirror adjustment – a first in the executive segment

Telematics

· Optional 7-inch (178 mm) LCD touch-screen display system, employing state-of-the-art fibre optics, for finger-tip control of climate, audio and navigation systems, and 


(where fitted) television tuner and dual-band, fixed GSM telephone

· Optional DVD-based satellite navigation system with post code destination 

programming - simply 'input' the postcode of the required destination and 

the satellite navigation automatically programmes the optimum route

· Optional dual-band, 8 Watt fixed GSM phone, operated by the handset, console 

switch panel, multi-function steering wheel controls, touch screen or voice 

activation, where fitted
· Optional JaguarVoice voice activation system for audio, phone and climate control 

systems, plus  – for the first time on S-TYPE navigation system and – TV tuner, 

where fitted. Now available in five languages plus N. American and Australian 

accents

· Standard 140 Watt, four-speaker Alpine sound system

· Optional 320 Watt, ten-speaker Alpine Premium Sound System with Digital 

Signal Processing optimising equalisation for three different seating combinations.

· The system includes two enclosed sub woofers with integral amplifiers.
· Trip computer with message centre standard on all models

Safety Technology

· Standard-fit Adaptive Restraint Technology System (A.R.T.S.) with ultrasonic front passenger sensing

· Energy-absorbent seating system incorporating an anti-whiplash mechanism in the front seat back rests

· Front and rear pyrotechnic buckle pre-tensioners
· Side curtain airbags for front and rear occupants 

· Side airbags for driver and front passenger

· Decoupling brake pedal mechanism

· Stiffer, stronger body exceeds crash legislation worldwide

· Intrusion beams fitted to front and rear doors

· Dual-stage frontal airbags for driver and front passenger

· Auto headlights

· Deformable plastic fuel tank

· New anti-trap windows all round and sunroof, where specified

Security

· Thatcham-approved, Category 1 remote control locking/alarm system with 


perimeter alarm and immobiliser

· Ultrasonic intrusion sensing 

· Fully encrypted, electromechanical steering column lock responds only to dedicated transponder key 

· Global open/close for windows and sunroof

· 2 stage unlocking

Exterior Design and Engineering

· Stiffer, stronger body with a 10 per cent increase in torsional rigidity

· Lightweight, high-strength magnesium front seat frames, facia structure and 

cross-car beam
· Bench-style rear seat with 60/40 split, based on new lightweight, high-strength 

steel frame – except S-TYPE R where the rear seat has a structural 'shear ring' 

which provides a further 7 per cent improvement in torsional stiffness

· ‘Freeform’ reflector technology halogen headlamps

· Xenon high-intensity discharge (HID) low-beam projector headlamps are standard  on S-TYPE R (available across the S-TYPE as an option), and produces more than twice the light of a halogen system. 

· Electronic headlamp levelling and automatically operating headlamps, responding 

to ambient light

· A new front grille with an integrated Jaguar ‘growler’ emblem

· New exterior mirrors (with optional electrochromatic function)

· Front grille, integral with front bumper, moves backwards in minor accidents 

without damaging adjacent components 

· Fine-wire heated wiper park fitted as standard to front screen (full heated screen 

is an option)

· Rear screen contains antenna element for audio system and, where fitted, TV tuner
· Reverse Park Control (option)

· Rain-sensitive wipers (option)

NEW S-TYPE RANGE IN DETAIL

POWERTRAIN

New engines widen customer choice

Three new engines, including the first applications of Jaguar’s new 4.2 litre  AJ-V8 in both naturally aspirated and supercharged forms, widen customer choice and enlarge the breadth and the depth of the Jaguar S-TYPE range.

· The new 400 horsepower S-TYPE R brings a supercharged engine to 

Jaguar’s S-TYPE saloon range for the first time - the most powerful series production 

engine ever manufactured by Jaguar. Joining the acclaimed Jaguar XJR and XKR, the

STYPE R takes the number of current supercharged models to three.

· The naturally aspirated version of the new 4.2 litre engine offers increased levels 

of performance and refinement over the previous 4.0 litre.

· An all-aluminium 2.5 litre AJ-V6 extends the S-TYPE’s accessibility and affordability. 

It slots in below the existing 3.0 litre AJ-V6, which itself now offers improved 


  driveability.

Developed at Jaguar’s Whitley Engineering Centre (Coventry, England), all three new engines deliver refined performance and driveability, in keeping with the company’s traditions. In addition, a new six-speed ZF automatic transmission optimises the responsiveness of both 4.2 litre engines. The same transmission is also available with the new 2.5 litre and 3.0 litre engines.

New 2.5 litre AJ-V6 engine – class-leading power

Combining outstanding driveability with class-leading power for a naturally aspirated 2.5, Jaguar's 24-valve, 2.5 litre AJ-V6 engine delivers  a maximum power of 201 bhp DIN (150 kW DIN) at 6800 rev/min.

New to the S-TYPE range, the efficient four-cam 2.5 litre engine delivers spirited, refined performance and competitive fuel economy. The S-TYPE 2.5 manual has a 0-60 mph time of 8.2 seconds (0-100 km/h – in 8.6 seconds), a top speed of 142 mph (228 km/h) and a combined fuel economy figure of 29.6 mpg (9.5 l/100km). 

Maximum torque of 250 Nm DIN (184 lb ft) DIN is achieved at 4000 rev/min. Over 90 per cent of peak torque is available between 2,500 and 6,000 rev/min, which endows the S-TYPE 2.5 with outstanding flexibility. A major contributor to this exceptionally wide torque spread is the continuously Variable Cam Phasing system, which adjusts the timing of valve opening and closing, depending on engine speed, load and oil temperature. This allows engine performance to be optimised for excellent mid-range torque, improved full-load performance, reduced emissions and improved fuel economy.

The 60-degree AJ-V6 engine has a lightweight but exceptionally stiff structure, which makes a major contribution to the engine’s overall refinement. It also has a forged steel crankshaft with four main bearings, two overhead chain-driven cast iron camshafts per bank and four valves per cylinder, with direct-acting mechanical bucket tappets. Other key features include a four-stage tuned intake manifold and state-of-the-art, 32-bit Denso electronic engine management system. 

The Jaguar S-TYPE 2.5 litre is available with a Getrag five-speed manual transmission 

or with the new, six-speed ZF automatic.

Improved 3.0 litre AJ-V6 engine

Like its new 2.5 litre stablemate, the established S-TYPE 3.0 litre AJ-V6 delivers a class-leading output for its size of normally aspirated engine. Maximum power is 240 bhp DIN (179 kW DIN) at 6800 rev/min and maximum torque 300 Nm DIN (221 lb ft) DIN at 4100 rev/min.

The revised 3.0 litre offers enhanced driveability, as well as a fuel economy improvement of eight per cent –  27.4 mpg (10.3 l/100 km) for a manual car on the combined cycle. The new 3.0 litre model achieves the 0-60 mph sprint in 7.3 seconds (0-100 km/h – 7.6 seconds) and a top speed of 146 mph (235 km/h).

Key to the more responsive driving characteristics is a flatter torque curve, achieved by the improved three-stage variable geometry induction system and revised Variable Cam Phasing (VCP). Now continuously variable, the VCP also includes a new vane-type system – shared with the 2.5 litre engine - for quicker engine response and increased refinement. 

The Jaguar S-TYPE 3.0 is available with a Getrag five-speed manual transmission or 

the new, six-speed ZF automatic.

New 4.2 litre AJ-V8 naturally aspirated engine - refined power

The new, 32-valve, 4.2 litre naturally aspirated AJ-V8 develops 300 bhp (298 PS/219 kW EEC or 224 kW DIN) at 6000 rev/min, an increase of seven per cent on the acclaimed 4.0 litre AJ-V8 it replaces. Torque also improves throughout the rev range and now peaks almost eight per cent higher developing 411 Nm (308 lb ft) EEC or 420 Nm (310 lb ft) DIN at 4100 rev/min.

The 4196cc, four-cam engine, with its standard-fit six-speed ZF automatic gearbox, takes the S-TYPE from 0-60 mph in 6.2 seconds (0-100 km/h in 6.5 secs) and on to a top speed (limited) of 155 mph (250 km/h). Refinement is enhanced, fuel economy benefits and emissions performance also improve.

The increased displacement and longer stroke of the 4.2 improves driveability and increases power throughout the rev range, with particular benefit at lower engine speeds. The already high compression ratio is increased to 11:1 (from 10.5:1), benefiting combustion efficiency. The inlet manifold structure is also optimised for improved response.  

New, branched inlet manifolds increase the engine’s performance throughout the range, by improving air/fuel distribution to the eight cylinders, arranged in 90 degree ‘Vee’ configuration. This complements enhancements to the combustion system, to reduce cold start emissions and improve low emission/high economy calibrations.  

A more responsive, vane-type design is introduced for the Variable Cam Phasing (VCP), which optimises the camshaft timing, to help deliver high torque across the rev range, improve idle stability and reduce emissions. 

Other developments within the engine include:

· New crankshaft

· Revised lubrication system

· Revised exhaust system

· Modified cylinder heads

· Revised engine management system

· Improved camshaft drive system

Overall engine refinement benefits from a new, ultra-quiet drive chain and improved acoustic damping. Inherent strengths of the AJ-V8 design retained for the new 4.2 litre engine include the extremely compact, lightweight and rigid cylinder block that ensures refinement and efficiency. 

Other features include a sophisticated, 32-bit engine management system, a crankshaft supported by five main bearings and two chain-driven overhead camshafts per bank activating four valves per cylinder via direct-acting bucket tappets.  

New 4.2 litre AJ-V8 supercharged engine - ultra high performance

With an exhilarating 400 bhp (DIN) on tap (395 PS/281 kW EEC or 298 kW DIN) 

at 6100 rev/min, Jaguar's new, supercharged 4.2 litre AJ-V8 engine makes the new 

S-TYPE R one of the most powerful cars in its class. The supercharged engine delivers a massive 553 Nm (408 lb ft) DIN or 538 Nm (397 lb ft) EEC of torque at 3500 rev/min – 32 per cent higher than the naturally aspirated 4.2 engine. More importantly, the massive torque of the supercharged engine delivers blistering in-gear performance, with instantly accessible acceleration for safe overtaking.

The inherent stiffness of the AJ-V8 structure provides an excellent platform for refined pressure charging. The mechanically-driven Eaton supercharger delivers excellent boost characteristics, which combined with twin intercoolers - results in outstanding performance under all driving conditions. For the 4.2 litre application, the supercharger speeds are increased by 5 per cent. At only 1300 rev/min, the supercharged engine is already producing more torque than the naturally aspirated engine at its peak. From 1500 rpm in excess of 80% of torque is always available.

The supercharged 4.2 AJ-V8 engine shares all the mechanical advances of its new, naturally aspirated sibling, with consequent benefits for performance, economy and refinement.

Special developments for the supercharged version include forged pistons with 

oil-cooling jets, high fin-density intercoolers, and a bespoke twin fuel pump system.  The air inlet system is also revised, with twin inlets for the air box and low-loss inlet trunking and outlet ducts on the supercharger. The normally aspirated has one inlet. 

The new supercharged 4.2 AJ-V8 is only available in S-TYPE R, where it is matched 

to the new six-speed ZF automatic transmission. 

State-of-the-art ZF six-speed automatic transmission

With the introduction of the extended engine line-up, the Jaguar S-TYPE becomes 

the first car in its class to offer the state-of-the-art ZF 6HP26 six-speed automatic transmission (across the range).  

This delivers refined, accessible performance with seamless gearchanges and 

world-class quality and reliability. It is standard on both supercharged and naturally aspirated 4.2 litre engines, and is available across the new S-TYPE line-up, including 

the AJ-V6 2.5 litre entry model.

The new transmission employs a planetary gear train principle using Lepelletier-type gear sets. This facilitates six forward gears with a dramatically reduced number 

of components: 470 parts compared with 660 in the S-TYPE’s previous five-speed automatic representing a weight saving of around 8kg. The system offers a wide gear ratio spread and a higher torque capacity to satisfy the exacting performance requirements for off-the-line acceleration, a high maximum speed and excellent fuel economy across all S-TYPE models. 

The new six-speed automatic is controlled by a Bosch ‘Mechatronic’ brain – an integrated electro-hydraulic control module, housed as a single unit inside the transmission. The control module ensures the quality and consistency of gear shifts, as each module’s ECU (electronic control unit) is calibrated to its own unique hydraulic control body during the manufacturing process.

Using microhybrid printed circuit board technology, the ECU of the electro-hydraulic module is compact and lightweight. Enclosed in a sealed metal housing, the ECU remains totally impervious to the high temperatures and vibration that result from 

the unit being located inside the transmission. With its significantly reduced number 

of components and electrical connections, the ‘Mechatronic’ control module also increases the overall reliability and robustness of the automatic transmission.

The new gear train system and the use of plastic for the transmission’s oil sump and the body of the ‘Mechatronic’ control module, reduces weight by around 11 kilograms compared with the previous five-speed automatic transmission.

Adaptive gearshift strategies for improved response

Unique to the S-TYPE R, the electronic brain of the new 6-speed ZF automatic transmission incorporates adaptive strategies which improve the accessibility of the vehicle's performance in demanding driving conditions while maintaining a relaxed driving experience when cruising.

In 'Sport' mode, throttle pedal usage and cornering behaviour are monitored to assess driving style and road conditions. When an enthusiastic driving style or a demanding road is detected, 6th gear is inhibited and the lower gears are made slightly more accessible in order to prevent unwanted 'hunting' between gears. Conversely, when cruising conditions are detected, 6th gear is once again made available to maximise driving refinement and economy.

Under conditions of heavy braking, the transmission will perform one or more downshifts to improve response to a subsequent throttle pedal application. Similarly if the throttle pedal is released rapidly following hard acceleration, one or more upshifts are inhibited to increase engine braking and also improve subsequent response. 

To complement these features, when a corner is detected transmission upshifts are inhibited. This inhibition is also maintained for a short distance after the corner allowing the driver to achieve a smooth balance through the bend without unwanted shifting mid-corner.

New electronic ‘J-gate’ gear selector 

Jaguar's unique ‘J-gate’ gear selector system is adapted to suit the new six-speed transmission and now features electronic control, contributing to smoother shift operation and ergonomics. Key features include: 

· Shift-by-wire (on manual side of the J-gate – gears 2, 3, 4 and 5) 

· Weight reduction (magnesium mounting frame)

· Improved mounting position for better craftsmanship and NVH

· Improved setting procedure  (aided by new ZF transmission) 

· New design of gear shift interlock mechanism

On the right-hand side of the selector gate, the driver is presented with the traditional ‘PRND’ layout which, in the ‘D’ position, provides fully automatic availability of all six gears. When moving the lever to the left hand side of the ‘J-gate’, sixth gear is not selectable, providing the driver with a ‘5 4 3 2’ gear range selection.

A mode switch adjacent to the ‘J-gate’ allows selection between normal and sport shift modes. When the sport mode is selected, the gear shift points are extended to make full use of the engine’s power.

Five speed Getrag manual transmission 

The Getrag Type 221, five-speed manual transmission is available with both the 2.5 

and 3.0 litre AJ-V6 engines.  It features a direct drive fifth gear and a 3.07:1 final drive ratio, enhancing fuel economy and engine refinement.

VEHICLE DYNAMICS

Chassis refinements for enhanced vehicle dynamics

The vision for Jaguar's chassis engineers was threefold:

· 
Further enhance suspension refinement and the ride and handling balance, 

with improved roll control and steering feel

· 
Further enhance the ‘sporting feel’ of the S-TYPE chassis

· 
Develop the suspension systems to suit the ride and handling requirements 


   of the new high performance, supercharged S-TYPE R

"The new S-TYPE has very connected, linear and positive steering, never nervous, so the car inspires confidence under all conditions and speeds.  The driving dynamics are a combination of agility and stability.  These are often conflicting requirements but this car is very stable, it's fun to drive on twisting roads and safe and secure at high speed."

MIKE CROSS, CHIEF ENGINEER, VEHICLE INTEGRITY

New front and revised rear suspensions, retuned springs, dampers, bushes and anti-roll bars reinforce S-TYPE’s reputation for outstanding ride comfort and poised, agile handling.  

S-TYPE's stiff body also contributes to a smooth, stable ride by providing a rigid mounting for suspension components such as springs, dampers and bushes, allowing them to work to optimum efficiency. This helps to isolate the effects of road irregularities, minimising the transmission of noise and vibration into the cabin. It also contributes to an overall feel of robustness in all driver interfaces, including steering, pedals and instruments.

The body of the S-TYPE R features a unique bolt-in structural 'shear ring' around the rear seat back, which provides a further 7 per cent improvement in torsional stiffness. This benefits both body rigidity and vehicle handling in this ultra high performance, high torque, supercharged S-TYPE saloon.   

New front and revised rear suspensions

The all-new front suspension system with aluminum control arms includes:

· New double wishbones of different lengths to minimise effect of changes 


in track and camber

· Lower wishbone with two individual arms joined by a ball joint

· Inclination of the upper control arm axis providing improved anti-dive 

· New upper A-frame control arm to body, aluminium forging, gives revised geometry 


        for improved straight-ahead stability. Control arm houses two new fluid block 

                                 bushes and integral ball joint for improved refinement 

· New two-piece aluminium forged lower control arms Lateral arm locates damper, 

        anti roll-bar drop link and knuckle, and connects via a rubber bush to a separate 

(non-isolated) sub-frame

· Forward arm locates via a ball bush to the lateral arm and via a hydraulic 


 bush to the subframe for improved vibration damping 

· New stiffened swan neck knuckle, aluminium squeeze casting with integral steering 


arm, press fit lower ball joint, locates disc shield (riveted) and brake calliper, 

generation 3 wheel bearing with integral ABS

· Unique knuckle to accommodate supercharged V8

· New peripheral steel front sub-frame, non-isolated, locating lower control arm 


mountings, anti roll-bar, cooling module, steering rack and engine hydro-mounts.

· Retuned springs and dampers

· Tubular design anti-roll bars with revised routing to accommodate supercharger 

package and new suspension

Changes to the rear suspension include:

· New subframe, with mounts moved outboard to edge of longitudinals for increased

stiffness

· Double-shear bracket, increasing stiffness of rear frame mounting to body for 

improved refinement and handling

· Larger diameter bushes (hydraulic on front face of frame) for enhanced ride refinement

· New toe-link for improved rear wheel alignment

· Lowered roll-centre to maintain balance with new front suspension

· New tubular larger diameter anti-roll bar

· New lower control arms and knuckle to accommodate increased torque of  

supercharged S-TYPE R

Class-leading body stiffness benefits chassis dynamics

The S-TYPE’s ultra-stiff body, now 10 percent stiffer contributes to a smooth, stable ride, by providing a rigid mounting for suspension components such as springs, dampers and bushes. Thanks to this, the components work to optimum efficiency, isolating the effects of road irregularities with little or no transfer to the body. 

By enabling the suspension components to work to maximum effect, the S-TYPE’s body stiffness plays a key role in minimising noise and vibration, as well as eradicating potential squeaks and rattles. It also contributes to an overall feel of robustness in 

all driver interfaces, including steering, pedals and instruments.  

On the V6 and V8 Sport models, steering and suspension components such as springs, dampers and steering rack are specially tuned for even tauter, more responsive handling.  

Electronic Parking Brake (automatic transmission) – an Industry-first in the executive segment

The Electronic Parking Brake provides both ease of use and space saving benefits.  

The parking brake is operated by a console-mounted switch. The brake is applied automatically whenever the ignition key is removed and released whenever the gear selector is moved from the ‘park’ position.

Calliper levers are applied via a cable mechanism through the parking brake actuator mounted on the rear sub-frame. The electronic control module (ECU) is mounted in the boot. The driver receives an audible and visual warning if the parking brake is accidentally applied while the vehicle is in motion.

Brakes – designed for high performance

The braking system on the new S-TYPE features large diameter, ventilated front and solid rear discs, a four channel anti-lock system and panic assist braking, together with Electronic Traction Control and Dynamic Stability Control system as standard.  

Steel-braided brake hoses, which are durable yet lightweight, provide reduced expansion under pressure and reduced permeability compared with rubber hoses.

The front brake system of all models except the R incorporate twin piston, aluminum sliding callipers and ventilated discs (30mm width by 300mm diameter on all models except V6 Sport and V8). At the rear, all models except the R have 20 mm x 288 mm discs with a single-piston, cast iron sliding calliper.   

To suit the dynamic handling characteristics of the Sport models and the increased power of the naturally aspirated 4.2 litre V8 engine, larger diameter (320 mm) ventilated front discs increase thermal capacity and provide improved brake feel.  

The S-TYPE R, with its supercharged 4.2 litre V8 engine, features 32 x 365 mm ventilated front discs and 15 x 330 mm solid rear discs, with four-piston aluminum brake calipers developed in conjunction with racing brakes specialist Brembo.

A double-diaphragm vacuum booster and electrically driven vacuum pump ensure 

that the discs and callipers operate at maximum efficiency, and contribute to easy, responsive pedal feel under all driving conditions

Anti-lock braking system with Panic Brake Assist 

The new S-TYPE is equipped as standard with a sophisticated anti-lock braking system.  The system is four-channel, with independent, digital inputs from sensors in all four wheels, and incorporates Panic Brake Assist (PBA). 

PBA helps the driver in an emergency when it senses maximum braking power isn’t being used. Often in emergency braking, the driver does not press the brake pedal hard enough. PBA senses emergency braking from the speed at which the driver hits the brake pedal, and, at that precise moment, applies maximum available braking power. This is achieved by new travel and pressure sensors on the booster and brake master cylinder.

Dynamic Stability Control now standard on all models, controlling both oversteer and understeer

The Dynamic Stability Control system (DSC), fitted as standard on all S-TYPE models, detects and counteracts the effects of both understeer and oversteer. The system employs electronic sensors to calculate actual vehicle motion and compares it with the direction initially chosen by the driver. If the system perceives that the driver’s intended direction does not match that of the vehicle, it intervenes by applying brake force to any required combination of the four wheels.

DSC relies on the speed of communication between many different systems, via the vehicle’s multiplexed electrical architecture. The system’s electronic control unit (ECU) receives data from a number of sensors, including: a steering angle sensor, which calculates the driver’s intended direction; a yaw rate sensor, which calculates oversteer/understeer and side slip (actual vehicle motion); and the four wheel-speed sensors, which also form part of the anti-lock system. In addition, a pressure transducer provides brake pressure feedback and an engine torque interface feeds torque data back to the ECU.

The DSC system incorporates all new hardware: 

· 
New (Mark 25) modulator and ECU with increased processing power, integrating anti-lock, panic brake assist and traction control systems. 

· Active wheel speed sensors

· 
Steering rate sensors (located on steering column) to calculate steering angle 

· 
Yaw rate sensor and lateral accelerometer to calculate vehicle oversteer / understeer and vehicle side slip

· Wheel speed sensors (ABS Sensors)

· 
Primary and secondary pressure transducers (located on brake master cylinder) for pressure feedback

· Active booster – replaces passive booster – to supply brake pressure without driver input

Computer Active Technology Suspension (CATS) – system enhancement

Jaguar’s sophisticated CATS system optimises ride and handling by using uprated springs to increase roll stiffness and two-stage adaptive dampers under electronic control for ride refinement. The CATS system is very similar to that fitted to Jaguar 

XK8 and the V8 XJ Series saloon range.

The CATS dampers have two sets of characteristics, governed by an electronic control unit (ECU). Sensor inputs are provided by vertical accelerometers on the bulkhead and in the luggage compartment, a lateral accelerometer within the under bonnet engine control module housing and a brake switch. The ECU automatically adjusts the damper settings in microseconds to suit the driving conditions, optimising ride and handling.

The result is a unique combination of agile handling, outstanding ride comfort and driver control. The operation of the CATS system is completely automatic, requiring 

no intervention from the driver. CATS is available as an option pack, together with 

7.5 x 17 Herakles Sport alloy wheels.

CATS is a two-stage, adaptive damping ride control system that switches the dampers from their normal ‘soft’ to ‘firm’ setting and back, when certain pre-determined road and vehicle dynamic conditions are met. 

The control module uses the information from the vehicle speed and accelerometers and compares this with a pre-programmed set of tuned parameters in the software to determine the appropriate damper setting.

When the system is functioning normally, the driver does not have any visual indication when the damper settings are changing from firm to soft.

System Benefits

· CATS automatically optimises ride and handling under all conditions, maximising comfort, handling and driver control

· Provides refined ride comfort for normal driving

· 
Automatically changes all four dampers to stiffer sports handling when road conditions demand 

· 
Automatically changes to stiffer sports handling when cornering, as with the original system, but now allows the inner wheels to return to softer settings mid-bend, giving improved ride comfort and increased tractability from the rear wheels

· Automatically changes to stiffer sports handling when braking. CATS now allows the dampers to return to their soft setting earlier by continually monitoring the brake switch and instantaneous deceleration rate, unlike the original system, which remains stiff regardless of the deceleration rate during the remainder of the stop. This reduces the harshness of the road inputs during braking events

· Automatically switches the rear dampers slightly ahead of the front pair at low speed to reduce transient understeer, giving improve agility and turn-in performance

· 
Automatically switches the front pair ahead of the rear pair at high speed to increase transient understeer, giving improved stability

NEW INTERIOR DESIGN AND ENGINEERING

"The interior of the new S-TYPE harmoniously blends the latest technology 

and safety engineering with hand-crafted natural forms and surface materials. The result is a luxurious, functional and contemporary environment, with a distinctive Jaguar character."

IAN CALLUM, DIRECTOR OF DESIGN

New interior combines Jaguar craftsmanship with the latest technology

The S-TYPE’s interior is enhanced to offer even greater comfort and superior levels of luxury. A redesigned facia, centre console and door panels create an interior which combines Jaguar quality and craftsmanship with leading-edge technology and the latest innovations in safety engineering.

Built on a strong but exceptionally lightweight magnesium base frame structure, the new facia features a luxurious, full-width bird’s eye maple wood panel. In the middle of the facia, a redesigned centre console houses the audio and climate control systems, in addition to a range of optional multimedia and communications systems. These include a 7-inch (178 mm) widescreen LCD touch-screen display and a DVD-based satellite navigation system.

Across the range, attention to detail is reflected in new steering column switchgear, window switches and optional glass sunroof, which now all feature one-touch activation and anti-trap protection.

The subtle, undulating curves of the facia are complemented by the bold design and crisp detailing of the instrument cluster. Sharing a common family theme with the 

X-TYPE, the instrument gauges, with their distinctive 'racing green' dials and slender pointers, reinforce the sporting feel, craftsmanship and quality of the S-TYPE's enhanced interior. Bright-finish detailing, for example on door handles and speaker surrounds, adds a finishing touch to the luxurious interior.

The centre console, which runs down from the facia between the front seats, now offers improved comfort and convenience, culminating in a wide, sliding front centre armrest with dual cupholders shrouds a convenient cubby box. The bird’s eye maple facia design is harmoniously carried through to the door panels, enhancing the sumptuous yet relaxed ambience of the 

S-TYPE’s interior.

Electronically adjustable pedals with memory for the optimum driving position

The optional electronically adjustable pedal system enables the optimum driving position to be obtained, in conjunction with seat and steering wheel adjustment.  

The pedals can be moved backwards or forwards using a switch on the steering column and can be programmed with two memorised positions. The pedals will automatically revert to position 1 or position 2 when a switch located in the driver's door panel is pressed or, if programmed into the remote key fob, when the car is unlocked.

The system incorporates new throttle, brake and clutch pedal assemblies, including pedal adjustment drive motors and cables. 

A brake pedal decoupler mechanism provides added protection, on all models, for the driver's feet and legs in the event of a severe frontal impact.

New front seats with light weight magnesium frames

New seats are also introduced to provide increased comfort, with Classic, Sports 

and R performance front seat designs specially developed to suit specific models 

and specification levels. SE models are equipped with electrically adjustable front 

seats as standard, featuring 8-way driver and 6-way passenger adjustment, while the 

R performance model features 16-way electric adjustment. All front seat designs are 

built around a strong but exceptionally lightweight magnesium frame. Driver comfort can be further enhanced with optional electrically adjustable foot pedals linked to automatic memory function, the first system of its kind available on a production car.

All S-TYPE models benefit from a new 140 Watt Alpine-sourced audio system, featuring a radio cassette head unit, which can be upgraded to an optional Mini Disc or single CD, to suit customer preferences. Customers can also specify the optional 320 Watt Jaguar premium sound system, with ten speakers and an optional six-disc CD autochanger located in the boot. The standard system is also compatible with the optional six-disc CD autochanger as a stand-alone option.

S-TYPE is now available with six interior trim colour options, four of which are new.  Dove and Sand have been introduced across the range, in addition to Heritage Tan 

and Cranberry exclusively available on Sport and R models.

TELEMATICS

Class-leading telematics systems

The class-leading telematics and communications technologies, originally introduced 

on X-TYPE, are now available as options on the new S-TYPE range. These include a 

7-inch (178 mm) widescreen LCD touch-screen display, employing thin film transistor technology (TFT), providing finger-tip control of climate, audio and navigation 

systems, and (where fitted) television tuner and dual-band, fixed GSM telephone.  

This exceptionally high level of integration is facilitated by fibre optics to provide 

fast, high-quality data transfer.

JaguarVoice – more languages

The optional JaguarVoice voice activation system, a world-first for Jaguar when launched on S-TYPE in 1999, provides voice control of primary functions of the audio, telephone and climate control systems, as well as - for the first time on S-TYPE - the satellite navigation system and TV tuner, where fitted. Maintaining Jaguar’s lead in the field of voice activation, this system is now available in French and Spanish, as well as the languages that were available at launch: German, Italian and English, including North American and Australian accents.

DVD Sat Nav with postcode programming

The optional DVD satellite navigation system, replacing the previous CD Rom-based system, is carried over from X-TYPE and features post-code entry destination programming. The operator simply inputs the postcode of the required destination 

and the navigation system plots a route and guides them. 

DVD technology enables approximately eight times the amount of information to be stored on one disc compared to conventional CD-ROM based systems and delivers highly accurate information, virtually instantaneously. The navigation system features 'voice guidance', now selectable in nine languages and, where optional voice activation is fitted, voice-activated 'name tags' in three languages. Navigation mapping coverage includes 17 major markets.

TV tuner

The television tuner receives all freely available terrestrial channels, with stereo 

TV audio, widescreen picture options and teletext services. TV pictures will only 

be displayed when the vehicle is stationary. When the vehicle is moving, the system can provide TV audio whilst, for example, displaying navigation information on the display.

Dual-band, fixed GSM phone

The S-TYPE's premium specification Motorola dual-band, fixed GSM phone, available 

as a factory fit option, was originally introduced by Jaguar as the world's first in-car telephone of its kind on Jaguar XJ, XK and Daimler models for 2001 model year and subsequently on the X-TYPE range.

Dual-band technology offers the user a choice of networks and provides optimum phone performance and call quality by automatically and seamlessly selecting either the 900MHz or 1800MHz frequency bands, depending on local conditions.  

With 8 Watts of power, the phone has a far stronger signal than a 2-Watt portable system, a particularly valuable feature in areas where network coverage is weak or at times when network congestion is high. When using an 8-Watt, fixed phone under these conditions, the call is far less likely to be lost.  

The new unit offers a number of useful features, including:

· Via the optional voice activation system, the phone will recognise up to 40 numbers using voice-tags such as "home" and "office", and automatically dial the stored number

· A filtering system which cancels out background noise, for enhanced hands-free operation

· 
Speech encryption, for confidentiality and security

· 
A memo recording facility, with up to three minutes' memory

· 
A data port, allowing information to be transferred to and from a laptop PC at 14.5 KB per second

· 
The capability to display and compose Short Message Service (SMS) text messages via the touch-screen, if fitted

The telephone is fully integrated with the car's audio system and can be operated via the handset, switch panel, multi-function steering wheel or touch-screen, if fitted.  Hidden neatly away in the centre console, the phone preserves the luxurious look of the car's interior and discourages theft. The antenna is concealed within the rear bumper.  

Optional 320 Watt, ten-speaker Alpine Premium Sound System

For enhanced in-car entertainment, a single-slot CD player and a single-slot MiniDisk (MD) player are available as options. In addition a 320 Watt, ten-speaker Alpine Premium Sound System may be specified as an option to the standard 140 Watt, 

four-speaker system. A 6-disc CD autochanger is available as an option.

The Alpine Premium Sound System features Digital Signal Processing, creating a 'virtual stage' inside the car at the touch of a button, optimising equalisation for three different seating combinations: driver, front passenger or all occupants. The system features four lightweight mid-bass, door-mounted speakers, four door-mounted tweeters and two enclosed sub woofers with integral amplifiers, packaged beneath the rear parcel tray.

A trip computer with message centre – standard fitment on all models – displays detailed journey data as well as information on the functioning of a wide range of vehicle systems, including security and engine diagnostics.

SAFETY TECHNOLOGY 

Additional safety equipment including Adaptive Restraint 

Technology System (A.R.T.S.) and side curtain airbags

The S-TYPE’s already comprehensive occupant safety systems – including driver and passenger front and side airbags – are further enhanced with the introduction of side curtain airbags for front and rear occupants, energy-absorbent front seat backrests and rear seat belt pre-tensioners. In addition, Jaguar’s leading-edge Adaptive Restraint Technology System (A.R.T.S.), is now standard fitment across the S-TYPE range.  

Originally introduced by Jaguar on XK Series sports cars in 2001, A.R.T.S. represents a major advance in passive vehicle safety. A.R.T.S. employs the world’s most comprehensive and sophisticated ultrasonic sensing technology to monitor the position of the front passenger, while an array of other sensors detect the front passenger’s weight, the position of the driver in relation to the steering wheel, the usage of seat belts and, should a crash occur, the severity of the impact. 

Armed with this information, the inherent flexibility of Adaptive Restraint Technology System (A.R.T.S.) provides optimum protection, tailored to the individual needs of each front seat occupant and the specific characteristics of an impact. In particular, the system greatly reduces the risk of airbag-related injuries caused by inappropriate airbag deployments.

For rear impact protection, both front seats have an active energy-absorbent seating system, which with correctly positioned headrests, are designed to reduce the risk 

of neck and spinal whiplash injuries in the event of a rear impact by controlling the rearward motion of the seat back. In a rear-end collision, depending on the severity 

of the impact, occupant weight and occupant posture, the front seat backrest moves rearwards and reclines in a controlled motion, minimising the relative motion of the head and torso.

EXTERIOR DESIGN AND ENGINEERING

Subtle exterior refinements

Detail refinements to the exterior of the S-TYPE subtly enhance the car's distinctively stylish design.  Exterior styling plays a key role in influencing the purchasing decision, and S-TYPE's styling has proved highly desirable to customers ever since the car's launch.

A subtly changed grille, featuring an integrated Jaguar ‘growler’ emblem, is the main visual change to the front of the car. Along the sides, new chrome window surrounds on entry and SE models further emphasis the S-TYPE’s flowing lines, enhancing the car’s distinctive profile. In addition, exterior mirrors (with optional electrochromatic function), a range of new alloy wheel designs and new badging contribute to the subtle exterior detail refinements.

"S-TYPE’s styling maintains Jaguar’s long tradition of producing some of the most beautiful, innovative and distinctive saloon car designs in the world.  With the new range, we took the opportunity to incorporate subtle exterior styling enhancements, building on a successful design that has proved extremely successful with our customers. The interior of the new S-TYPE harmoniously blends the latest technology and safety engineering with 

hand-crafted natural forms and surface materials. The result is a luxurious, functional and contemporary environment, with a distinct Jaguar character."

IAN CALLUM, DIRECTOR OF DESIGN

Xenon HID headlamps option

Xenon high-intensity discharge (HID) low-beam projector headlamps are available on the new S-TYPE.  Xenon headlamps produce more than twice the light of a halogen system.  Performance is further enhanced by uniform illumination across the beam pattern, with a colour similar to that of natural daylight.  Road markings and traffic signs become more highly visible and the dentification of potential hazards can be recognised at an earlier stage, helping to make night driving safer and more relaxing. Xenon headlamps are standard on the S-TYPE R and an option across the rest of the range.

This option comes complete with telescopic headlamp power wash and automatic, dynamic headlamp levelling, which continuously adapts the beam level in accordance with driving conditions.  

Body enhancements

The new S-TYPE's body, now 10 per cent stiffer, is a highly stable structure, delivering significant benefits in terms of strength, durability, robustness and refinement. A stiff body structure ensures highly consistent door-to-body shut lines and door seal location, benefiting both cosmetic appearance and vehicle refinement. S-TYPE's one-piece body side with integral rear wing and one-piece front door opening panel are now complemented by a laser-welded roof joint for increased stiffness, strength and joint quality. The already extensive use of high strength steels in critical areas of the body is increased for the new model range with the addition of ultra high strength steels, including sills, A pillars, B-C pillars and cantrails for weight optimisation, enhanced structural integrity and to accommodate side-curtain airbags. 

S-TYPE's stiff body also contributes to a smooth, stable ride by providing a rigid mounting for suspension components such as springs, dampers and bushes, allowing them to work to optimum efficiency. This helps to isolate the effects of  road irregularities, minimising the transmission of noise and vibration into the cabin. It also contributes to an overall feel of robustness in all driver interfaces, including steering, pedals and instruments.

The body of the S-TYPE R features a unique bolt-in structural 'shear ring' around the rear seat back, which provides a further seven per cent improvement in torsional stiffness. This benefits both body rigidity and vehicle handling in this ultra high performance, high torque, supercharged S-TYPE saloon.   

MANUFACTURING

S-TYPE is manufactured at Jaguar’s plant at Castle Bromwich, Birmingham, England, where state-of-the-art production facilities and leading-edge logistics practices combine to ensure the highest standards of quality and efficiency.

Development of the Castle Bromwich plant, which began with the approval of the original S-TYPE programme, represented one of the largest UK inner city investments for 50 years. The development process included a three-phase programme to extend, refurbish and re-equip the paint shop, which commenced in 1996 and was completed in 2001.

‘Lean’ logistics

To keep inventory to a minimum, thus avoiding inconvenient and potentially costly stockpiling, the majority of components are delivered directly to line-side on a strict ‘just-in-time’ basis. Typically, only those parts on long-haul delivery consignments from distant countries such as Japan are stored, and usually no more than ten days’ worth of such stock is held.

Line-side, the objective is to hold no more than two hours’ stock. To achieve this, 

the latest techniques in ‘Kanban’ radio data telemetry are utilised. Kanban, originally pioneered in Japan, is a scheduling method which disciplines product replenishment  by suppliers, based on customer usage.

Since the launch of S-Type in 1999, the Castle Bromwich plant has continued to implement the latest principles of 'lean’ logistics and is considered to be a highly effective implementation with significant benefits to inventory reduction.

Body shop, paint shop and final assembly – at a glance

The S-TYPE is constructed, painted and finally assembled into a complete car on the same site.  (Bodies for Jaguar XJ saloon and XK sports cars are constructed and painted at Castle Bromwich and then shipped to Jaguar's Browns Lane plant in Coventry for final assembly).

Body shop

· 
The latest-generation, computer-controlled robots apply 4,500 spot welds (75 per cent of all welds) on each S-TYPE body, using closed-loop secondary current control for precision welding

· Robot MiG (Metal in inert Gas) welding and plasma braze jointing methods are also used

· There are 142 robots and 130 body shop workers per shift, of whom 20 are equipment maintenance specialists

· 
The one-piece body side pressings, incorporating the rear wing, nearly 4 metres long, are formed in a 2,000 tonne capacity press

· 
Door and bonnet outer panels are made from a special bake-hardening steel, more resistant to minor impacts

· Accurate fitting of doors, bonnet and bootlid is ensured by precise measurement  of body openings, achieved by means of a laser camera system, which also performs a final check on the overall geometry of the body

Paint shop

· 
The paint shop is a sealed, clean room with controlled temperature and humidity.   Among the environmental measures are close control and incineration of remaining paint solvent emissions

· 
A new, environmentally efficient, gas-fired boiler house and effluent treatment facility were installed at Castle Bromwich for S-TYPE production

· 
A waterborne paint system cuts basecoat solvent emissions by 85 per cent

· 
The body is internally protected by hot wax injection, which flows into place during stoving

Final assembly

· 
Final assembly is performed on a series of skillets (platforms of laminated wood) to create a continuously moving floor for efficient, ergonomic assembly.

· 
The complete driveline assembly is dynamically balanced in a special rig, rotating the propshaft at 2500 rev/min

· 
When mating the driveline to the body, laser cameras are used to validate alignment

· 
Each completed car is the subject of a comprehensive electrical check by means of  VCATS (Vehicle Configuration And Test Systems)

· 
Non-contact laser technology measures front and rear suspension geometry and headlamp alignment

· 
Further validation is performed on a rolling road, including engine, transmission and brakes

· 
At the final inspection stage, each car passes through a high-intensity water spray line, equivalent to a monsoon, tilting the car to simulate ‘awkward’ parking situations. All water test facilities feature a low-loss re-circulation system

· 
The exit door to the despatch compound will only open if the car’s individual electronic ‘record card’ indicates that every single process has been validated

	
	2.5 V6
	2.5 V6
	2.5 V6 
	3.0 V6
	3.0 V6
	4.2 V8
	S-TYPE R

	
	
	Sport
	SE
	Sport
	SE
	
	(4.2 V8 S/C)

	 TRANSMISSION/SUSPENSION
	
	
	
	
	
	
	
	

	 Drivetrain control
	-Dynamic Stability Control
	(
	(
	(
	(
	(
	(
	(

	
	-traction control
	(
	(
	(
	(
	(
	(
	(

	 Suspension
	-standard
	(
	�
	(
	�
	(
	(
	�

	
	-sports
	�
	(
	�
	(
	�
	�
	�

	
	-CATS (Computer Active Technology Suspension), 

only available with 17”or 18” wheels 
	(O)
	O
	(O)
	O
	(O)
	O
	�

	
	
	
	
	
	
	
	
	

	
	-R Performance suspension including CATS
	�
	�
	�
	�
	�
	�
	(

	 Transmission
	-5-speed manual
	(
	(
	∆
	(
	∆
	�
	�

	
	-6-speed electronic automatic
	O
	O
	(
	O
	(
	(
	(

	 EXTERIOR FEATURES
	
	
	
	
	
	
	
	

	 Chrome finish grille surround, boot plinth, bumper blades and window surrounds
	(
	�
	(
	�
	(
	(
	�

	 Body coloured grille surround, boot plinth, bumper blades and black window surrounds
	�
	(
	�
	(
	�
	�
	(

	 Body coloured rear spoiler
	�
	O
	�
	O
	�
	�
	(

	 Exterior mirrors 
	-power-adjustable, heated
	(
	(
	(
	(
	(
	(
	(

	
	-electrochromic, interior and door mirrors
	O
	O
	(
	O
	(
	(
	(

	
	-power fold back
	O
	O
	(
	O
	(
	(
	(

	 Front fog lights 
	O
	(
	(
	(
	(
	(
	(

	 Garage door opener
	O
	O
	O
	O
	O
	O
	O

	 Headlights
	-automatic headlights
	(
	(
	(
	(
	(
	(
	(

	
	-headlight powerwash
	O
	O
	O
	O
	O
	O
	(

	
	-manual headlight levelling
	(
	(
	(
	(
	(
	(
	�

	
	-xenon headlights with automatic headlight levelling (1)
	(O)
	(O)
	(O)
	(O)
	(O)
	(O)
	(

	 Heated front windscreen
	O
	O
	O
	O
	O
	O
	O

	 Heated rear window with timer
	(
	(
	(
	(
	(
	(
	(

	 Metallic paint 
	O
	O
	(
	O
	(
	(
	(

	 Rain-sensing windscreen wipers (only available with electrochromic mirrors)
	(O)
	(O)
	(O)
	O
	O
	O
	O

	 Reverse park control
	O
	O
	O
	O
	O
	O
	O

	 Sunroof 
	-glass, electric with tilt and slide
	O
	O
	O
	O
	O
	O
	O

	 Wheels
	-cast alloy 6.5” x 16” Helios
	(
	�
	�
	�
	�
	�
	�

	
	-cast alloy 7.5” x 16” Spirit 
	O
	�
	(
	�
	(
	�
	�

	
	-cast alloy 7.5” x 16” Artemis
	O
	�
	O
	�
	O
	�
	�

	
	-cast alloy 7.5” x 17” Kronos
	�
	�
	�
	�
	�
	(
	�

	
	-cast alloy 7.5” x 17” Sport
	O
	O
	O
	O
	O
	O
	�

	
	-cast alloy 7.5” x 17” Herakles
	�
	(
	�
	(
	�
	�
	�

	 
	-cast alloy 8” x 18”/9.5”x 18” Zeus
	�
	�
	�
	�
	�
	�
	(

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	


( Standard    O  Optional at extra cost    ∆ Optional at no extra cost    � Not available    (O) Linked option

	
	
	2.5 V6
	2.5 V6
	2.5 V6 
	3.0 V6
	3.0 V6
	4.2 V8
	S-TYPE R

	
	
	
	Sport
	SE
	Sport
	SE
	
	(4.2 V8 S/C)

	 EXTERIOR FEATURES CONTINUED
	
	
	
	
	
	
	
	

	 Wheels cont... 
	-two-piece alloy 8” x 18” Indianapolis (champagne finish)
	O
	O
	O
	O
	O
	O
	�

	 
	-two-piece alloy 8” x 18” Indianapolis (silver finish)
	O
	O
	O
	O
	O
	O
	�

	 
	-two-piece alloy 8” x 18” Monaco (champagne finish)
	O
	O
	O
	O
	O
	O
	�             

	 
	-two-piece alloy 8” x 18” Monaco (silver finish)
	O
	O
	O
	O
	O
	O
	�

	
	-spare space saver
	(
	(
	(
	(
	(
	(
	(

	
	-spare full size (16” or 17”)
	O
	O
	O
	O
	O
	O
	�

	 INTERIOR THEME, TRIM AND SEATING
	
	
	
	
	
	
	
	

	 Classic interior theme
	-colour keyed upper facia, door top rolls and carpets, Bronze stained birds eye maple veneer facia and door trims
	(
	�
	(
	�
	(
	(
	�

	 Sport interior theme
	-Warm Charcoal upper facia and door top rolls, flint carpets,  

grey stained birds eye maple veneer facia and door trims with Sport seats 
	�
	(
	�
	(
	�
	�
	�

	 R interior theme
	-Warm Charcoal upper facia and door top rolls, flint carpets,  grey stained birds eye maple veneer facia and door trims with R Performance seats and gearknob 
	�
	�
	�
	�
	�
	�
	(

	 Steering wheels
	-leather
	(
	�
	∆
	�
	∆
	∆
	�

	
	-sport leather
	�
	(
	�
	(
	�
	�
	�

	
	-R leather
	�
	�
	�
	�
	�
	�
	(

	
	-wood/leather
	O
	�
	(
	�
	(
	(
	�

	 Gearknobs
	-leather (manual)
	(
	(
	∆
	(
	∆
	�
	�

	
	-sport leather with black metal sides (automatic)
	�
	(O)
	�
	(O)
	�
	�
	�

	
	-R leather with chrome sides
	�
	�
	�
	�
	�
	�
	(

	
	-wooden (automatic)
	O
	�
	(
	�
	(
	(
	�

	 Seats, front
	-electrically-adjustable, driver and front passenger
	(
	(
	(
	(
	(
	(
	(

	
	-heated
	O
	O
	O
	O
	O
	O
	O

	 Classic seats, cloth  
	-6-way driver, 4-way passenger 
	(
	�
	�
	�
	�
	�
	�

	
	-10-way driver, 8-way passenger
	O
	�
	�
	�
	�
	�
	�

	 Classic seats, leather  
	-8-way driver, 6-way passenger 
	O
	�
	�
	�
	�
	�
	�

	
	-8-way driver with memory pack, 6-way passenger  
	O
	�
	(
	�
	(
	(
	�

	
	-10-way driver with memory pack, 8-way passenger  
	O
	�
	O
	�
	O
	O
	�

	
	-16-way driver with memory pack, 16-way passenger  
	�
	�
	O
	�
	O
	O
	∆

	 Sport seats, leather (perforated) 
	-8-way driver, 6-way passenger 
	O
	(
	�
	(
	�
	�
	�

	
	-10-way driver with memory pack, 8-way passenger 
	O
	O
	O
	O
	O
	O
	�

	 R Performance seats, leather  two tone) 
	-16-way driver with memory pack, 16-way passenger
	�
	�
	�
	�
	�
	�
	(

	 R Performance seats, leather (monotone 
	-16-way driver with memory pack, 16-way passenger
	�
	�
	�
	�
	�
	�
	∆

	 Ivory leather 
	
	(O)
	O
	O
	O
	O
	O
	∆


( Standard    O Optional at extra cost   ∆ Optional at no extra cost    �   Not available    (O) Linked option

	
	
	2.5 V6
	2.5 V6

Sport
	2.5 V6 

SE
	3.0 V6

Sport
	3.0 V6

SE
	4.2 V8
	S-TYPE R

(4.2 V8 S/C)  

	 INTERIOR THEME, TRIM AND SEATING CONTINUED

	 Seats, rear 
	-head restraints x 3 and Isofix seat brackets
	(
	(
	(
	(
	(
	(
	(

	
	-fold down seat (split 60/40)
	(
	(
	(
	(
	(
	(
	�

	 INTERIOR FEATURES
	
	
	
	
	
	
	
	

	 Air conditioning
	-automatic climate control with air filtration, dual  
	
	
	
	
	
	
	

	
	zone for driver and passenger temperature control
	(
	(
	(
	(
	(
	(
	(

	 Cruise control 
	-with steering wheel audio and telephone controls
	O
	O
	(
	O
	(
	(
	(

	 Cupholders
	-front (with powerpoint) 
	O
	O
	(
	O
	(
	(
	(

	
	-rear (only available with leather seats) 
	(O)
	O
	O
	O
	O
	O
	O

	 Foot pedals
	-electrically-adjustable
	O
	O
	O
	O
	O
	O
	O

	 Footwell mats 
	-carpet, front and rear
	O
	O
	O
	O
	O
	O
	(

	 Power-assisted steering
	-speed-sensitive, variable-ratio
	(
	(
	(
	(
	(
	(
	(

	 Ski hatch with bag (only available with leather seats) 
	
	(O)
	O
	O
	O
	O
	O
	(

	 Steering column 
	-electrically-adjustable with tilt away
	(
	(
	(
	(
	(
	(
	(

	 Sunblind
	-rear, electric
	O
	O
	O
	O
	O
	O
	O

	 Sunvisors
	-illuminated with vanity mirror, driver and passenger
	(
	(
	(
	(
	(
	(
	(

	 Trip Computer
	-with twin function
	(
	(
	(
	(
	(
	(
	(

	 AUDIO/COMMUNICATIONS
	
	
	
	
	
	
	
	

	 Audio equipment
	-Jaguar sound system with 4 twin-cone speakers, 

EON RDS, PTY, TA and automatic volume control,  CD compatible
	(
	(
	(
	(
	(
	(
	(

	
	-single slot CD (a)
	O
	O
	O
	O
	O
	O
	O

	
	- CD autochanger (6-disc capacity) (b)
	O
	O
	O
	O
	O
	O
	O

	
	- Mini Disc (c)
	O
	O
	O
	O
	O
	O
	O

	
	- Jaguar Premium sound system with 10 speakers, DSP, 
	
	
	
	
	
	
	

	power amplifier  
	(only available with a, b, or c options above)
	(O)
	(O)
	(O)
	(O)
	(O)
	(O)
	(O)

	 JaguarVoice/cruise control
	-voice control of audio, automatic climate control and,   
	
	
	
	
	
	
	

	
	  if fitted, telephone and navigation (with cruise control)
	(O)
	(O)
	O
	(O)
	O
	O
	O

	 Navigation system (2)
	-touch screen 
	O
	O
	O
	O
	O
	O
	O

	 Navigation/TV system (2)
	-touch screen and TV tuner
	O
	O
	O
	O
	O
	O
	O

	 Telephone system 
	-with integrated GSM  
	(O)
	(O)
	O
	(O)
	O
	O
	(O)


( Standard    O  Optional at extra cost    ∆ Optional at no extra cost    � Not available    (O) Linked option

	
	2.5 V6
	2.5 V6

Sport
	2.5 V6 

SE
	3.0 V6

Sport
	3.0 V6

SE
	4.2 V8
	S-TYPE R

(4.2 V8 S/C)

	 SAFETY
	
	
	
	
	
	
	
	

	 Adaptive Restraint Technology System (A.R.T.S.)
	(
	(
	(
	(
	(
	(
	(

	 Airbags
	- front and side for driver and front passenger
	(
	(
	(
	(
	(
	(
	(

	
	- side curtain
	(
	(
	(
	(
	(
	(
	(

	 Anti-lock braking system
	(
	(
	(
	(
	(
	(
	(

	 Emergency brake assist
	(
	(
	(
	(
	(
	(
	(

	 Energy absorbent seating system
	(
	(
	(
	(
	(
	(
	(

	 First aid kit
	O
	O
	O
	O
	O
	O
	O

	 High-level stop light
	
	(
	(
	(
	(
	(
	(
	(

	 Seat belt pre-tensioners
	- front and rear
	(
	(
	(
	(
	(
	(
	(

	 Warning triangle 
	
	O
	O
	O
	O
	O
	O
	O

	 SECURITY
	
	
	
	
	
	
	
	

	
	- security system with central locking, ultrasonic 

intrusion-sensing, radio frequency remote control 

and engine immobiliser (Thatcham 1 approved)
	(
	(
	(
	(
	(
	(
	(

	 OPTIONAL PACKS
	
	
	
	
	
	
	
	

	 Memory pack 

 (see seating for availability)
	-memory for driver’s seat, (with entry/exit function), 

door mirrors, steering column and adjustable foot pedals  (if fitted) 
	(O)
	(O)
	(
	(O)
	(
	(
	(

	 Winter pack 
	-headlight powerwash, heated front windscreen,  heated front seats
	O
	O
	O
	O
	O
	O
	�


( Standard    O  Optional at extra cost    ∆ Optional at no extra cost    � Not available    (O) Linked option

(1)Only available with headlight powerwash.

(2)Country mapping DVD covers:  Austria, Belgium, Denmark, France, Germany, Italy, Luxembourg, Netherlands,Portugal*, Spain*  Sweden (South), Switzerland and the United Kingdom. * Limited coverage

Specifications vary from country to country.

Note: 2.5 V6 SE and 3.0 V6 SE are known as Executive in Ireland.

	
	2.5 V6 /

2.5 V6 Sport
	2.5 V6

SE
	3.0 V6

SE
	3.0 V6

Sport
	4.2 V8
	S-TYPE R 

(4.2 V8 S/C)

	ENGINE

	Cylinders/valves per cylinder
	6/4
	6/4
	6/4
	6/4
	8/4
	8/4

	Bore/stroke – mm
	81.65/79.5
	81.65/79.5
	89.0/79.5
	89.0/79.5
	86.0/90.3
	86.0/90.3

	Capacity – cc
	2497
	2497
	2967
	2967
	4196
	4196

	Maximum power – DIN bhp (kW) 
	201 (150)
	201 (150)
	240 (179)
	240 (179)
	300 (224)
	400 (298)

	EEC PS (kW)
	200 (147)
	200 (147)
	238 (175)
	238 (175)
	298 (219)
	395 (291)

	@ rev/min
	6800
	6800
	6800
	6800
	6000
	6100

	Maximum torque – DIN lb.ft. (Nm)
	184 (250)
	184 (250)
	221 (300)
	221 (300)
	310 (420)
	408 (553)

	EEC lb.ft. (Nm)
	181 (245)
	181 (245)
	216 (293)
	216 (293)
	303 (411)
	399 (541)

	@ rev/min
	4000
	4000
	4100
	4100
	4100
	3500

	Compression ratio :1
	10.5
	10.5
	10.5
	10.5
	11.0
	9.1

	Transmission
	5-speed
	6-speed
	6-speed
	5-speed
	6-speed
	6-speed 

	
	manual
	automatic
	automatic
	manual
	automatic
	automatic

	PERFORMANCE (MANUFACTURER’S ESTIMATES)
	
	
	
	
	
	

	Acceleration 0-60 mph (0-100 km/h) – sec
	8.2 (8.6)
	9.4 (9.9)
	7.5 (7.9)
	7.3 (7.6)
	6.2 (6.5)
	5.3 (5.6)

	Top speed – mph (km/h)
	142 (228)
	140 (225)
	145 (233)
	146 (235)
	155 (250) Limited
	155 (250) Limited

	FUEL CONSUMPTION (1)
	
	
	
	
	
	

	Urban – mpg (l/100km) 
	21.2 (13.3)
	19.4 (14.5)
	18.5 (15.3)
	19.2 (14.7)
	16.5 (17.1)
	15.3 (18.5)

	Extra urban – mpg (l/100km)
	38.7 (7.3)
	35.8 (7.9)
	35.1 (8.0)
	36.7 (7.7)
	34.2 (8.2)
	30.5 (9.2)

	Combined – mpg (l/100km)
	29.6 (9.5)
	27.4 (10.3)
	26.4 (10.7)
	27.4 (10.3)
	24.5 (11.5)
	22.5 (12.5)

	Carbon dioxide emissions – g/km
	229
	249
	259
	249
	274
	314

	Tank capacity – gals (Ltr) approx.  (incl. 5 Ltr reserve)
	15.0 (69.5)
	15.0 (69.5)
	15.0 (69.5)
	15.0 (69.5)
	15.0 (69.5)
	15.0 (69.5)

	WEIGHT (2) 
	
	
	
	
	
	

	Kerb weight – lb (kg) 
	3572 (1620)
	3667 (1663)
	3771 (1710)
	3689 (1673)
	3826 (1735)
	3969 (1800)

	Gross vehicle weight – lb (kg)
	4690 (2127)
	4754 (2156)
	4806 (2180)
	4736 (2148)
	4839 (2195)
	4938 (2240)

	Permitted roof load – lb (kg)
	220 (100)
	220 (100)
	220 (100)
	220 (100)
	220 (100)
	220 (100)

	Permitted trailer load, unbraked – lb (kg) (3)
	1654 (750)
	1654 (750)
	1654 (750)
	1654 (750)
	1654 (750)
	1654 (750)

	Permitted trailer load, braked – lb (kg) (3)
	3528 (1600)
	4079 (1850)
	4079 (1850)
	4079 (1850)
	4079 (1850)
	4079 (1850)

	Permitted towbar download – lb (kg)
	165 (75)
	165 (75)
	165 (75)
	165 (75)
	165 (75)
	165 (75)


(1)The fuel economy ﬁgures were obtained in tests carried out in line with the Passenger Car Fuel Consumption Amendment Order 1996 (Reﬂecting EU Directive 1999/100 EEC fuel consumption test cycle). 

These ﬁgures were correct at the time of going to press.

(2)Please consult your dealer for details of changes associated with option ﬁtment. Kerb weights reﬂect vehicles to standard speciﬁcations.Optional extras increase weight.

(3)Up to maximum 12% gradient.

	
	2.5 V6 /
	2.5 V6
	3.0 V6
	3.0 V6
	4.2 V8
	S-TYPE R 

	
	2.5 V6 Sport
	SE
	SE
	Sport
	
	(4.2 V8 S/C)

	WHEELS/TYRES
	
	
	
	
	
	

	Wheel type
	Helios Alloy

6.5" x 16" /

Herakles Alloy

7.5 x 17”
	Spirit Alloy

7.5" x 16"
	Spirit Alloy

7.5" x 16"
	Herakles Alloy

7.5" x 17"
	Kronos Alloy

 7.5" x 17"
	Zeus Alloy

8.0" x 18" Front 

	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	9.5" x 18" Rear

	
	
	
	

	Tyre size
	205/60 ZR16 /

235/50 ZR17
	225/55 ZR16
	225/55 ZR16
	235/50 ZR17 
	235/50 ZR17
	245/40 ZR18 F

	
	
	
	275/35 ZR18 R

	Turning circle (kerb to kerb) – ft. in. (m)
	37’ 8" (11.48)
	37’ 8" (11.48)
	37’ 8" (11.48)
	37’ 8" (11.48)
	37’ 8" (11.48)
	37’ 8" (11.48)

	Boot volume – cu.ft. (litres – VDA) 
	14.1 (400)
	14.1 (400)
	14.1 (400)
	14.1 (400)
	14.1 (400)
	14.1 (400)

	Boot volume with rear seats folded – cu.ft. (litres – VDA)
	28.6 (810)
	28.6 (810)
	28.6 (810)
	28.6 (810)
	28.6 (810)
	N/A


[image: image2.png]TN

JAGUAR




Dimensions shown in mm.

NOTE: The dimension details refer to a nominal car but may vary marginally from car to car due to design and  production tolerances.  The nominal condition has been used wherever possible. Please consult your dealer for details of changes associated with option ﬁtment.

PRICE LIST

Model
List
VAT on
Retail
On-the-road


Price
List Price
Price
Price

2.5 litre V6
£20,634.04
£3,610.96
£24,245.00
£24,950.00

2.5 litre V6 Sport
£22,761.70
£3,983.30
£26,745.00
£27,450.00

2.5 litre SE
£23,995.74
£4,199.26
£28,195.00
£28,900.00

3.0 litre V6 Sport
£24,889.36
£4,355.64
£29,245.00
£29,950.00

3.0 litre V6 SE
£26,123.40
£4,571.60
£30,695.00
£31,400.00

4.2 litre V8 
£30,038.30
£5,256.70
£35,295.00
£36,000.00

4.2 litre V8 R
£39,740.43
£6,954.57
£46,695.00
£47,400.00

PRICE AND SPECIFICATION

All prices are based on manufacturer’s recommended prices. 

List Prices apply throughout Great Britain, Northern Ireland, the Channel Islands and the Isle of Man. “On the road prices” are included for the benefit of purchasers in Great Britain and Northern Ireland only and are calculated using Retail Prices from this price list and adding 12 months Vehicle Excise Duty, delivery to the Jaguar Dealer premises, first registration fee and an estimated price for number plates and a full tank of fuel. 

All Jaguar vehicles first registered after 01 March 2001 will attract Vehicle Excise Duty at £155 per annum (Band D Petrol).

Prices and specifications are subject to change without notice. 

Factory fitted options indicated on the original order will be supplied at extra cost. The prices are applicable to vehicles sold for permanent use in the UK.

WARRANTY 

Three year (60,000 miles)*

Vehicles included in this price list are covered by Jaguar Total Care which consists of the above warranty together with the following:

Emergency Assistance via RAC Motoring Services in the UK for three years

Mondial Assistance in Europe for three years

Paint Surface Warranty for three years

Corrosion (Perforation) Warranty for six years 

Optional extended warranties available:  

1.  4th year (100,000 miles)* 

2.  4th and 5th year (100,000 miles)*

3.  5th year (100,000 miles)* extension to 4th year

*Whichever occurs first after new vehicle purchase date. 

Please consult your Jaguar dealer for full details.







PAGE  

