Upper Arm RH 

Removal

- Remove the shock absorber and spring assembly.

- Remove the cabin air filter.

- Remove the cabin air filter housing retaining nut.

- Remove the cabin air filter housing.

- Remove the engine compartment support.

- Remove the engine compartment panel.

- Detach the wiring harness mounting bracket.

- Remove the upper arm retaining nut.

- Detach the engine compartment battery junction box.

- Release the engine compartment battery junction box retaining clips.

- Detach the engine compartment battery junction box.

- Detach the hydraulic control unit.

- Remove the upper arm retaining nut.

- Remove the upper arm.

Installation

- Install the upper arm.

- Install the upper arm retaining nut.

- Install the upper arm retaining nut.

- Install the shock absorber and spring assembly.

- CAUTION: The final tightening of the Upper arm retaining nut must be carried out with the vehicle on its wheels.

Tighten to 47 Nm .

- CAUTION: The final tightening of the Upper arm retaining nut must be carried out with the vehicle on its wheels.

Tighten to 47 Nm .

- Attach the hydraulic control unit.

- Attach the engine compartment battery junction box.

- reposition the engine compartment battery junction box .

- Attach the engine compartment battery junction box.

- Attach the wiring harness mounting bracket.

- Install the engine compartment panel.

- Install the engine compartment support.

- Install the cabin air filter housing.

- Install the cabin air filter housing retaining nut.

- Install the cabin air filter.