Shock Absorber and Spring Assembly 

Removal

- Remove the wheel and tire.

- Remove the stabilizer bar link retaining nut.

- Remove the stabilizer bar link.

- Detach the shock absorber.

- CAUTION: Make sure the wheel knuckle is supported. Failure to follow these instructions may result in damage to the vehicle.

Detach the upper arm.

- Lower the vehicle.

- Remove the shock absorber and spring assembly mounting cover plate.

- Disconnect the electrical connector.

- Remove the shock absorber and spring assembly.

Installation

- Install the shock absorber and spring assembly.

- Tighten to 27 Nm .

- Connect the electrical connector.

- Install the shock absorber and spring assembly mounting cover plate.

- CAUTION: Make sure the wheel knuckle is supported. Failure to follow these instructions may result in damage to the vehicle.

Attach the upper arm.

- Tighten to 90 Nm .

- Attach the shock absorber.

  - Tighten to 175 Nm .

- Install the stabilizer bar link.

- Attach the stabilizer bar link.

  - Tighten to 47 Nm .

- Install the wheel and tire assembly.

- Tighten to 70 Nm .