Service and Repair.

Drain - Raise and support the vehicle. - Place a suitable container under the transmission drain plug. - Remove the transmission drain plug. NOTE: If an integral problem is suspected, drain the transmission fluid through a paper filter. A small amount of metal or friction particles may be found from normal wear. However, if excessive metal and friction particles are present, internal examination may be required. - Install the transmission drain plug. Tighten to 45 Nm. Refill - Lower the vehicle. - Remove the automatic transmission fluid charging pipe cap. - Fill the automatic transmission with 8 liters of automatic transmission fluid through the automatic transmission fluid charging pipe. NOTE: Use automatic transmission fluid WSS-M2C922-A1 or equivalent meeting Jaguar specification. - Carry out a fluid level check and adjustment. See: Testing and Inspection

Capacity Specifications.

Transmission Fluid Capacity; 8.8 liters (9.3 quarts).

Fluid Type Specifications.

Automatic Transmission Fluid; ATF IDEMITSU K17 (Jatco 3100 PLO85).