Service and Repair.. Draining and Filling - Remove the engine roll restrictor mount retaining bolt. - Remove the transfer case mounting top retaining bolts. - Remove the transfer case mounting bottom retaining nuts. - Remove the transfer case mounting side retaining bolts. - Drain the transfer case oil. - Remove the transfer case pinion housing retaining bolts. - Remove the transfer case pinion housing. CAUTION: Transfer case oil will drain from the transfer case during this operation. NOTE: Observe the position of the shims between the pinion housing and the transfer case. Remove and discard the O-ring seal. - Install a new O-ring seal. - Install the pinion housing in the transfer case. - Install the pinion housing retaining bolts. - Install the pinion housing retaining bolts. - Tighten M8 studs to 35 Nm. CAUTION: Make sure the shims between the transfer case housing and pinion housing are correctly aligned. - Remove the transfer case fill plug. - Using a suitable syringe, add 550 Ml of SAE 75W 140 to the transfer case. - Install the transfer case fill plug. - Tighten to 20 Nm. - Install the engine roll restrictor bracket. - Tighten to 35 Nm. - Install the engine anti-roll restrictor bracket side retaining bolts. - Tighten to 35 Nm. - Install the engine anti-roll restrictor bracket top retaining bolts. - Tighten to 55 Nm. - Install right-hand catalyst..

Capacity Specifications.

Transfer Case Capacity; 0.6 liters (0.6 quarts).

Fluid Type Specifications.

Transfer Case Fluid; M2C 192A Synthetic.