Component Locations.

Relay and Fuse Location

Connector Locations.

Fuse Box Connectors

Fuse and Fusible Link Locations.

Fuse Details Engine compartment fuse box - fuses Fuse; Fuse; Rating; Colour; Circuit. No.; Type; (amps). F1; Cartridge; 20; Blue; Dip beam, High Intensity Discharge (HID) light relay.. F2; Cartridge; 50; Red; Ignition relay feed.. F3; Cartridge; 60; Yellow; ABS.. F4; Cartridge; 30; Pink; Rear window heater/heated mirrors relay.. F5; Cartridge; 30; Pink; Automatic transmission control module relay.. F6; Cartridge; 30; Pink; Heater blower relay.. F7; Cartridge; 50; Red; Power supply to passenger compartment fuse box.. F8; Cartridge; 80; Black; Blower module.. F9; Cartridge; 50; Red; Engine management system power relay.. F10; Cartridge; 20; Blue; Headlight power wash pump relay.. F11; Mini; Not used.. F12; Mini; Not used.. F13; Mini; 30; Green; Heated front screen right-hand side.. F14; Mini; Not used.. F15; Mini; 30; Green; Heated front screen left-hand side.. F16; Mini; 20; Yellow; Left-hand dip beam, day time running lights, autolights.. F17; Mini; 20; Yellow; Right-hand dip beam, HID lights.. F18; Mini; 20; Yellow; Throttle motor supply.. F19; Mini; 15; Blue; Screenwash pump, rain sensing system.. F20; Mini; 5; Tan; Navigation display.. F21; Mini; 15; Blue; Side (Parking) lights.. F22; Mini; 20; Yellow; Accessories relay.. F23; Mini; 10; Red; Air conditioning compressor clutch.. F24; Mini; 15; Blue; Door lock power.. F25; Mini; 15; Blue; Direction indicators (GEM).. F26; Mini; 15; Blue; Horns.. F27; Mini; 10; Red; Automatic transmission control module, transit relay.. F28; Mini; 15; Blue; Ignition switch.. F29; Mini; 30; Green; Starter motor solenoid.. F30; Mini; 30; Green; Accessory sockets.. F31; Mini; 10; Red; Engine management relay coil.. F32; Mini; 5; Tan; Engine management control.. F33; Not used.. F34; Diode D3; Not used.. F35; Diode D2; Engine management control relay diode.. F36; Mini; 10; Red; Canister purge valve, airflow meter, throttle motor relay, power wash relay, air conditioning relay.. F37; Mini; 15; Blue; Engine management system, blower speed controller, road speed sensor (if fitted).. F38; Mini; 30; Green; Oxygen sensor heaters A.. F39; Mini; 5; Tan; Variable intake valves.. F40; Mini; 15; Blue; Automatic transmission control module.. F41; Mini; 10; Red; Injectors.. F42; Mini; 30; Green; Oxygen sensor heaters B.. Passenger compartment fuse box - fuses Fuse; Fuse; Rating; Colour; Circuit. No.; Type; (amps). F60; Mini; 20; Yellow; Power windows left-hand.. F61; Mini; 20; Yellow; Power windows right-hand.. F62; Mini; 7.5; Brown; Rear fog lights.. F63; Mini; 7.5; Brown; Right-hand - front side light, side markers, rear tail lights.. F64; Mini; 7.5; Brown; Left-hand - front side light, side markers, rear tail lights.. F65; Mini; Not used.. F66; Mini; 30; Green; Left-hand power seat switches, heater module, lumbar pump.. F67; Mini; 5; Tan; Battery back-up sounder, transit relay, HID/dip relay, automatic transmission control module.. F68; Mini; 15; Blue; Cigar lighter.. F69; Mini; 7.5; Brown; Telematics display, In-car entertainment head unit, antenna module, navigation module, VICS (Japan), parking aid, amplifier premium 1 system, phone handset, phone power supply module, instrument cluster, PATS module.. F70; Mini; 30; Green; Right-hand power seat switches, heater module, lumbar pump.. F71; Mini; 15; Blue; VICS (Japan), telephone transceiver, telephone handset.. F72; Mini; 15; Blue; Radio head unit, CD player, navigation electronic control module, Jaguar diagnostic connector/OBDII connector.. F73; Mini; 15; Blue; Sunroof unit/module.. F74; Mini; 15; Blue; Instrument cluster, Premium ICE amplifier unit.. F75; Mini; 7.5; Brown; Mirror fold back module, battery back-up sounder, air conditioning ECU.. F76; Mini; Not used.. F77; Mini; 7.5; Brown; Left-hand and right-hand heated seats modules.. F78; Mini; 5; Tan; Phone power supply.. F79; Mini; 10; Red; Automatic transmission only: Reverse lights, reverse light switch, reverse park aid, electrochromic mirror, instrument cluster.. F80; Mini; 7.5; Brown; Park interlock, heater switch, OBDII, blower relay.. F81; Mini; 20; Yellow; Fuel pump control module.. F82; Mini; 10; Red; Variable power steering, alternator. Manual transmission only: Reverse lights, reverse light switch reverse park aid, electrochromic mirror, instrument cluster.. F83; Mini; 15; Blue; Not used.. F84; Mini; 5; Tan; Instrument cluster, passenger airbag fascia light, seat weight sensor.. F85; Mini; 7.5; Brown; Right-hand high beam light.. F86; Mini; 7.5; Brown; Left-hand high beam light.. F87; Mini; 15; Blue; Front fog lights, master light switch.. F88; Mini; Not used.. F89; Mini; Not used.. F90; Mini; 7.5; Brown; Stop lights, cruise control.. F91; Mini; 10; Red; Sunroof, electrochromic mirror, power windows, power mirrors.. F92; Mini; 10; Red; Manual transmission cruise control clutch switch, ignition coils, automatic transmission rotary switch.. F93; Mini; 30; Green; Windscreen wipers.. F94; Mini; 20; Yellow; HID module.. F95; Mini; 10; Red; Airbag restraints module.. F96; Mini; 7.5; Brown; ABS module.. F97; Mini; 7.5; Brown; Glove compartment light, instrument cluster lights, licence plate lights.. F98; Mini; 10; Red; Mirror heaters.. F99; Mini; 10; Red; Luggage compartment light, footwell lights, puddle lights, vanity mirror lights, interior lights, aircon control unit, roof console unit..