2010 Jaguar XF Dash Removal Notes (after lowering steering wheel in extended position and then disconnecting battery)

- Dash speaker grill (pushed in with multiple clips, 1 wiring harness plug;   an older grill will most likely break upon removal, I purchased a new one for around $140)

- Passenger side wood trim with buttons (pushed in with multiple clips and contains 1 wiring harness plug)

- Passenger side, plastic side panel (loosen rubber weather strip; pushed in with multiple clips)

- Driver side, plastic side panel (see #3)

- Passenger side silver vent trim (2 screws on side, 2 screws in front, 2 wiring harness plugs)

- Dash speaker (3 screws, 1 wiring harness plug)

- Driver side silver vent faceplate (pushed in with multiple clips)

- Instrument cluster plastic trim (top plastic cover, 3 screws in total on top, bottom left, bottom right)

- Driver side vent trim (2 screws on left, 2 screws on right, 1 wiring harness plug)

- Center silver vent trim (6 screws in total, 2 wiring harness plugs)

- Under glovebox carpeted panel (3 push together plastic clips)

- Glovebox (3 torx screws underneath, 2 hex screws front top, 1 hex screw on side, light wiring harness plug, 2 other wiring harness plugs, 1 ground screw from airbag)

- Passenger side suede pillar trim (1 torx screw under plastic cover)

- Driver side suede pillar trim (see #13)

- Center information screen (4 torx screws in front, 4 wiring harness plugs in back)

- Two plastic vent center pieces near windshield (allows access to dash screws)

- Driver side under dash carpeted panel (see #11)

- Driver side bottom panel (3 torx screws under, 2 hex screws top, 1 hex screw on side, 1 wiring harness plug)

- Spring loaded odometer cluster trim protector

- Odometer cluster (4 hex screws, 2 wiring harness plugs)

- Dash (12 hex screws in total including 3 driver side front, 3 passenger side front, 1 in center front, 1 driver side corner by door, 1 passenger side corner by door, 2 by windshield access through vent trim, 1 in speaker hole, 2 airbag wiring harness plugs)