EPB Lamp ON, Incorrect Park Brake Operation

1. Start the engine.

2. Firmly apply and release the footbrake five (5) times.

3. Confirm that the message 'NOT CALIBRATED' or 'APPLY FOOT AND PARK BRAKE' is displayed on the Instrument Cluster message center. This indicates that the parking brake is in calibration mode.

4. Lightly press the footbrake pedal.

5. Apply the parking brake by using the EPB switch.

6. Release the parking brake.

7. Release the footbrake.

8. Confirm that the brake warning lamp is no longer illuminated on the Instrument Cluster and that the 'NOT CALIBRATED' message is no longer displayed in the message center.

9. Apply and release the EPB five (5) times to ensure no error is present

You may need to repeat the footbrake/EPB switch 3 or 4 times.