Visor fix

My drivers side sun visor was hanging down about an inch from the roof, in addition whenever I folded it down and then put it back up it made a loud snap sound. Very unbecoming a Jaguar. Something had to be done. Searching on the web showed new ones at about $300.00 bucks - ouch! And finding a used one in my color, on the drivers side and not broken was proving to be challenging as well. So it was time to try to fix it myself.
And I am happy to report it took all of ten minutes and the fix is perfect and permanent. Steps to follow:
1.Get a flat screwdriver and pry mirror out. Just put in under the mirror around the four corners and it pops right out. (There are no wires behind the mirror, just contacts.
2. Pop off the cover holding the sun visor to the roof and get a torx bit ready to unscrew the bolt. You will need a bit of an extension because the space is tight (see what I fabricated in my pic)
3. Once the screw is out, pull out the whole thing from the roof and disconnect the wire.
4. Pull back the fabric and you will see the problem. The clam shell plastic housing is separated and needs to be tightly held together. I simply put in a one inch flat head dry wall screw to bring it back together tightly. (Maybe Gorilla glue would have worked as well?). Once I drilled it though the plastic I then removed the screw and used a Dremel to shave off the excess and then put it back in. It does not pierce the fabric on the other side. Put a little red X in the last pic, this is where the screw needs to be.
5. Put it all back together and you will see the sun visor now closes right up to the roof - nice and snug.
6.If the light on the mirror does not go on, press all four corners of the mirror down again, I guarantee the top part was not snapped back all the way in. The light will now work.

10 minutes of work and no cost!

Attached Thumbnails