jaghelp.com

Jaguar articles from a Jag specialist.

Wednesday, September 11, 2013

2005 XJ Fuse location and id

Everybody should have this fuse assortment in their cars in case you need fuses: ATD-364120 Piece Car Fuse Assortment

Engine compartment fuse box

| Fuse |  | Fuse |  | Rating | Colour |  | Circuit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Type |  | (amps) |  |  |

| F1 | ATO | 10A | Red | Front ignition, UHEGO, fuel pump, rear ignition relay coils |
| --- | --- | --- | --- | --- |
| F2 |  |  |  | Not used |
| F3 |  |  |  | Not used |
| F4 | ATO | 30A | Green | Powerwash pump |
| F5 |  |  |  | Not used |
| F6 | ATO | 40A | Orange | Air suspension compressor |
| F7 | ATO | 40A | Orange | Right-hand front screen heater or wiper park zone heater |
| F8 | ATO | 20A | Yellow | Right-hand dipped beam |
| F9 | ATO | 20A | Yellow | Left-hand dipped beam |
| F10 |  |  |  | Not used |
| F11 |  |  |  | Not used |
| F12 | ATO | 10A | Red | Engine control module |
| F13 | ATO | 10A | Red | Fuel injectors |
| F14 | ATO | 10A | Red | Purge valve, EGR valve, canister close valve (CCV), port deactivation, V6 intake manifold tuning valves (IMTV), MAFS, air filter solenoid, radiator fan controller |
| F15 |  |  |  | Not used |
| F16 | ATO | 20A | Yellow | Right-hand main beam, left-hand main beam |
| F17 | ATO | 10A | Red | Engine control module, transmission control module |
| F18 | ATO | 15A | Blue | Front fog lights, instrument cluster front fog status |
| F19 | ATO | 25A | Natural | Starter solenoid |
| F20 | ATO | 30A | Green | ABS module (pump) |
| F21 |  |  |  | Not used |
| F22 | ATO | 30A | Green | ABS module |
| F23 |  |  |  | Not used |
| F24 | ATO | 20A | Yellow | Throttle motor |
| F25 | ATO | 15A | Blue | Horns |
| F26 | ATO | 40A | Orange | Wiper system, front electronics module (FEM) |
| F27 |  |  |  | Not used |
| F28 |  |  |  | Not used |
| F29 |  |  |  | Not used |
| F30 | ATO | 10A | Red | Ignition coils |
| F31 | ATO | 15A | Blue | Intercooler water pump (supercharged engines) |
| F32 | ATO | 40A | Orange | Left-hand front screen heater |
| F33 | ATO | 30A | Green | 'A' bank heated oxygen sensors |
| F34 | ATO | 30A | Green | 'B' bank heated oxygen sensors |
| F35 | Midi | 80A |  | Radiator cooling fan |

Cabin fuse box

| F1 | ATO | 10A | Red | Climate control unit, fan, screen heaters and wiper relay coils, smog sensor |
| --- | --- | --- | --- | --- |
| F2 | ATO | 5A | Tan | Radiator fan controller, cruise control switch, adaptive cruise control |
| F3 | ATO | 5A | Tan | Rear outer seat, electrochromic mirror, driver seat heater, electric parkbrake, left and right-hand HID lamps levelling, left and right-hand seat belt switches |
| F4 | ATO | 5A | Tan | Instrument cluster |
| F5 |  |  |  | Not used |
| F6 |  |  |  | Not used |
| F7 | ATO | 5A | Tan | Cigar lighter, sunroof module, electric parkbrake switch illumination, accessory relays (trunk fuse box) |
| F8 | ATO | 5A | Tan | Instrument cluster |
| F9 | ATO | 10A | Red | Left-hand direction indicator and repeater |
| F10 | ATO | 5A | Tan | Glove compartment door motor and lamps |
| F11 | ATO | 5A | Tan | Radio muting (engine start), ignition feed |
| F12 | ATO | 10A | Red | Front electronic module (FEM), passenger mirror |
| F13 | ATO | 20A | Yellow | Rear electronic module (REM), door locks |
| F14 | ATO | 10A | Red | Courtesy light, footwell lamps, rain sensor, FEM |
| F15 | ATO | 5A | Tan | ECM, FEM, instrument cluster airbag fault lamp, instrument cluster ignition (run/start) |
| F16 | ATO | 5A | Tan | Diagnostic connector |
| F17 | ATO | 5A | Tan | ABS module |
| F18 |  |  |  | Not used |
| F19 | ATO | 5A | Tan | Ignition relay, ECM, REM, fuel pump relay, start relay |
| F20 | ATO | 5A | Tan | Telephone transceiver, TV module, VICS (Japan), voice module, rear telematics |
| F21 |  |  |  | Not used |
| F22 |  |  |  | Not used |
| F23 |  |  |  | Not used |
| F24 | ATO | 10A | Red | PATS transponder |
| F25 | ATO | 10A | Red | Right-hand direction indicator and repeater, right-hand front lamp, header console map lamp |
| F26 | ATO | 15A | Blue | Screenwash pump |
| F27 | ATO | 10A | Red | ACC module |
| F28 | ATO | 10A | Red | Climate control modules (front and rear), driver's door module (DDM) |
| F29 | ATO | 10A | Red | Airbag module, occupancy sensing module, passenger weight sensor |
| F30 | ATO | 10A | Red | Voice, front/rear sound, DVD, telematics, TV |
| F31 | ATO | 15A | Blue | Sunroof |
| F32 | ATO | 5A | Tan | Front/rear parking aid, trailer tow module, battery backed sounder, voice module, telephone receiver |
| F33 | ATO | 5A | Tan | Transmission control, 'J'-gate, generator |
| F34 | ATO | 5A | Tan | Navigation system |
| F35 | ATO | 10A | Red | Left-hand seat movement |
| F36 | ATO | 40A | Orange | Left-hand seat movement |
| F37 | ATO | 25A | Natural | Driver window |
| F38 | ATO | 15A | Blue | Radio head unit |
| F39 | ATO | 5A | Tan | Transit relay (removed before customer delivery) |
| F40 | ATO | 10A | Red | Security LED, yaw rate sensor |
| F41 | ATO | 20A | Yellow | Ignition switch |
| F42 | ATO | 10A | Red | FEM, REM, DDM, clock |
| F43 | ATO | 5A | Tan | Diagnostic connector |
| F44 | ATO | 5A | Tan | Brake switch |
| F45 | ATO | 10A | Red | Right-hand seat movement |
| F46 | ATO | 25A | Natural | Passenger window |
| F47 | ATO | 5A | Tan | Instrument pack |
| F48 | ATO | 20A | Yellow | Instrument pack (steering column tilt and reach) |
| F49 | ATO | 40A | Orange | Left-hand seat movement |

Trunk fuse box

| F1 |  |  |  | Not used |
| --- | --- | --- | --- | --- |
| F2 |  |  |  | Not used |
| F3 |  |  |  | Not used |
| F4 |  |  |  | Not used |
| F5 | ATO | 40A | Orange | Right-hand front seat |
| F6 | ATO | 10A | Red | Accessory connector |
| F7 | ATO | 40A | Orange | Front blower motor |
| F8 | ATO | 10A | Red | Heater circuit water pump |
| F9 | ATO | 25A | Natural | Rear seat motors |
| F10 |  |  |  | Not used |
| F11 | ATO | 5A | Tan | Rear footwell lamps, grab handle lamps |
| F12 | ATO | 10A | Red | Air suspension |
| F13 | ATO | 20A | Yellow | REM, rear seat heaters |
| F14 |  |  |  | Not used |
| F15 | ATO | 5A | Tan | Right-hand rear light cluster |
| F16 | ATO | 10A | Red | Trunk lights, driver seat switch pack, rear switch seat pack |
| F17 | ATO | 10A | Red | Left-hand rear fog and reversing lamps |
| F18 | ATO | 10A | Red | Number plate lamps, side marker lamps |
| F19 | ATO | 30A | Green | Heated rear window |
| F20 | ATO | 10A | Red | Heated door mirrors |
| F21 | ATO | 25A | Natural | Rear seat motors |
| F22 | ATO | 10A | Red | Accessory connector |
| F23 |  |  |  | Not used |
| F24 | ATO | 20A | Yellow | Fuel pump driver module |
| F25 | ATO | 15A | Blue | Front accessory socket |
| F26 | ATO | 15A | Blue | Rear accessory socket |
| F27 | ATO | 10A | Red | Rear CD |
| F28 | ATO | 25A | Natural | Left-hand rear window |
| F29 |  |  |  | Not used |
| F30 | ATO | 10A | Red | Generator, battery backed sounder, transit relay |
| F31 | ATO | 40A | Orange | Right-hand front seat |
| F32 | ATO | 30A | Green | Electric parkbrake actuator |
| F33 | ATO | 20A | Yellow | Rear electronic module (fuel pump) |
| F34 | ATO | 30A | Green | Power amplifier |
| F35 |  |  |  | Not used |
| F36 |  |  |  | Not used |
| F37 |  |  |  | Not used |
| F38 |  |  |  | Not used |
| F39 |  |  |  | Not used |
| F40 | ATO | 25A | Natural | Right-hand rear window |
| F41 |  |  |  | Not used |
| F42 |  |  |  | Not used |
| F43 | ATO | 15A | Blue | Rear cigar lighter |
| F44 | ATO | 15A | Blue | Front cigar lighter |
| F45 |  |  |  | Not used |
| F46 | ATO | 10A | Red | Right-hand rear reverse and fog lights |
| F47 | ATO | 20A | Yellow | FEM, front seat heater |
| F48 | ATO | 5A | Tan | High mounted stop lamps |
| F49 | ATO | 5A | Tan | Left-hand rear light cluster |
| F50 |  |  |  | Not used |
| F51 | ATO | 10A | Red | Navigation system |
| F52 | ATO | 20A | Yellow | Air suspension |
| F53 | ATO | 20A | Yellow | Rear fan motor |
| F54 | ATO | 15A | Blue | Heated steering wheel |
| F55 |  |  |  | Not used |