2008 Jaguar XJ 4.2L V-8. 113K miles

Air Suspension fault in message center.

Solid Yellow warning light in speedometer. Codes were C2303 and C2780. Car is level and does not sink down after 4 days. Rebuilt the air compressor and it's pumping fine.

Can clear C2303 but comes back after a short drive.

C2780 is an ASU error meaning ACU is in “Manufacturing Sub State” and needs reprogrammed.

Using the SDD I can program a new module for the ASU but after completing that the car goes directly to leveling task. I can’t find reprogram option in SDD just the new module option.

It runs the air pump off and on and seems to be leveling the car but it never completes inside SDD and I am stuck in an endless loop. Ran the leveling task for over an hour with no completion. Finally shut down SDD and the above errors are still present.

Would like the ASU reprogrammed and the car leveled if needed.

Note the car has an Arnott air strut on the front drivers side. It has a slight bit of oil on the outside of it but I can’t detect any leaking and the car sits level right now. I don’t think that is the problem?