[image: image1.png]Generic OBD Error Code Details for P0376

Timing Reference High Resolution Signal *

" Too Many Pulses

What does this mean?

OBD Code P0376refers to

The optical sensor provides a high resolution signal to the Powertrain Control Module (PCM) by counting puises on the sensor disk
located in the injection pump. The high resolution is one of the most important inputs by the PCM for fuel control and timing. This test
‘monitors the number of high resolution pulses which have been missed (not detected). Its based on a comparison between the
number of pulses that were detected since the last pump cam pulse and the number of the pulses that should have occurred. There
are approximately 64 high resolution pulses for every cam puise.

Symptoms. .
Possible sumptoms of OBD code P0376
- Engine Light ON (or Service Engine Soon Warning Light)

Causes

Possible causes of OBD code P0376

- Open or short in the optical sensor circuit - Optical sensor connector - Damaged optical sensor - Damaged PCM or ECM The
Error code is generally activated on detection of the following conditions: There are too many high resolution pulses from the optical
sensor to the PCM

Possible Solutions
- If damage, repair optical sensor circuit - Replaced optical sensor - Replaced PCM or ECM




