Adjusting Transmission to Throttle Cable relationship (Kickdown Cable)

To set up the TV cable or whatever you may want to call it.
Perform the following:

Throttle Wide open.

Pull the TV cable out as far as it would go.

Tighten adjusting nuts.

The idea being that the throttle cable and the TV cable must be able to work in unison with each other.  If at WOT you still have movement outwards on the TV cable, you have to adjust it to take up the slack.

WOT = TV cable out all the way. The proper geometry of the two cables already exist.

An incorrectly adjusted TV cable will cause considerable wear on the clutches and bands.

A loose TV cable will have the early shifts you are experiencing.

Tightening the TV cable will change the shift points. Too tight and you would have a harsh shift.

Once you get it where the throttle cable and the TV cable are working in unison, you can then tweak it to get the shift points where you want it.