TPS wiring my XJ-S HE with XJ40 TPS.

Before we start, my car has been completely REWIRED under the bonnet, so factory colours mean SQUAT, as does much of the factory electrical items that have either been removed, or relocated OUTSIDE the engine bay.
I also fitted a XJ40 TPS, just because, many, many years ago, and I will discuss the wiring of this using the TPS wire colours with just a small mention of my colours IF I feel it required, maybe not.

Now any TPS with 3 wires will work the car, surprised, DON’T BE. As long as the one you get rotates from low end to high end the same rotational way as the original, and this all occurs within 90deg. Waaaaay back when my Bourns died (RIP) I needed one in a hurry, and Jag were out of stock, hahaha, so I knicked one from a Mazda something. It powered the car for some time until I got the XJ40 unit and that is doing just fine.

The XJ40 TPS has 3 wires, Pink, Green, Black, simple.

On my TPS the Pink is at one end of the unit, the Green is the centre wire, and the Black is the other end.

The factory plug has 3 wires, surprise surprise, and 2 of them have 2 wires each, coz they pick up the coolant temp sensor, and air temp sensor going back to the ECU.

ALL these readings are with the ignition ON, and engine OFF, why waste petrol.

The readings were done with the meter RED lead as the probe, and the meter BLACK lead attached to earth.

The TPS PINK is the 5v (or very close) feed FROM the ECU, and NO change from idle to WOT.
The TPS GREEN is 0.39v at idle and 4.9v at WOT (full throttle).

The TPS BLACK is 0.05v at both idle and WOT.

The probing for the 0.34v +/- is via the TPS Black and Green wires.

I trust this is of some assistance to someone out in Jaguar land.
