Instrument Panel and Console - Glove Compartment

Removal and Installation

Removal

1. Remove the instrument panel end trim panel.

 Release the 3 clips.

2. CAUTION: Removal of the clips is a delicate

procedure, damage will occur if any force is used.

Remove the passenger side footwell trim panel.

- Remove the 3 clips.

3. Release the glove compartment.

              - Remove the 5 Torx screws.

              - Release the 2 clips.

4. Remove the glove compartment.

              - Disconnect the 2 electrical connectors.

              - Release the glove compartment lamp assembly.

.

Installation

1. Install the glove compartment.

 Install the lamp assembly.

 Connect and secure the electrical connectors.

 Tighten the Torx screws.

2. Install the passenger side footwell trim panel.

 Carefully secure the clips.

3. Install the instrument panel end trim panel.

 Release the door aperture seal.

 Carefully secure the clips.

 Install the aperture seal.