D2B Fiber Optic Cable Info

When I installed my phone I had a spare D2B connector terminated with a loop that was marked REAR ICE (In Car Entertainment.) If the car had the rear seat DVD this would have plugged into the other spare D2B cable marked BT INLINE. If you have this cable you don't need a new harness. The order on the D2B ring is:

Head Unit --> CD Changer --> Phone --> Voice --> Rear Media --> Nav --> Amplifier --> Head Unit

So if there is a spare connector for the rear media you can use it for the voice and everything is still in the proper order. If not, you need a new harness. I believe you can also add a connector and FO cable to the existing harness to create a new connection, but I haven't tried that.

Jaguar D2B ring diagnostic check
Press AM/FM + eject 
When the screen changes to the setup screen press 
eject + mix to cycle through the SW versions. 
I'd expect you should see:
MAIN 
CDC 
PHON 
VOIC - if you have voice rec 
NAVI 
If it doesn't cycle through these then it shows the D2B ring is broken

__________________________________________________ ______________
Next check to see that optional extras are all turned on.

Jaguar Diagnostics using Nav System Display
To get to the PIN entry menu hold in Phone and Menu for five seconds.
The code you want is 1971 - that will give you a MISC button that lets you turn on or off the optional extras like CDC, MD, phone, voice, etc.