To check for error codes in the Climate Control System with the internal diagnostics via the ‘Panel Codes’

1. Ignition off - hold down AUTO and RECIRCULATION buttons and
switch on the ignition
2. press AUTO button - if ZERO shows, there's no stored
error codes
3. press FACE button repeatedly to scroll through stored
fault codes
4. press FACE and HEATED REAR WINDOW buttons together to
clear a fault code.
5. repeat step 4 if there's more than one code
6. press FAN button to exit Interrogation Procedure
. 
Control Panel Fault Code Listing 
Reference fault code #23*: In ambient temperatures below 0°C, this code may be logged because the low ambient causes a temporary low gas pressure. Where the ambient temperature rises above 40°C, with the engine close to overheating, electrical feed to the compressor clutch may be cut and code #23 registered.
0 Normal operation no fault codes present None Wait 30 seconds for system self-check.
11 Motorized in-car aspirator malfunction Harness / connector fault Sensor open / short circuit Panel fault codes are not stored for motorized in-car aspirator motor failure.
12 Ambient temperature sensor malfunction Harness / connector fault Sensor open / short circuit After rectification, disconnect the vehicle battery for 10 seconds to reset the system.
13 Evaporator temperature sensor malfunction Harness / connector fault Sensor open / short circuit Refer to PDU
14 Water temperature input malfunction Instrument cluster output Refer to PDU
15 Heater matrix temperature sensor malfunction Harness / connector fault Sensor open / short circuit Refer to PDU
21 Solar sensor Sensor open /short circuit Refer to PDU
22 Compressor lock signal fault Low refrigerant charge, low compressor oil level, loose drive belt Harness / connector fault Adjust items as required
23 Refrigerant pressure switch malfunction Harness / connector fault Switch open / short circuit Refer to PDU23 Refrigerant pressure low refrigerant charge *Leak from damaged pipe or joint Rectify as required and recharge system
24 Face vent demand potentiometer fault Potentiometer open / short circuit Harness / connector fault Refer to PDU
31 LH fresh / recirc. potentiometer fault Harness / connector fault  In certain circumstances the servo motor may over-travel and cause further logged faults. This may be cured, following fault rectification, by cycling the ignition ON-OFF-ON 3 times Refer to PDU
32 RH fresh / recirc. potentiometer fault Harness / connector fault  In certain circumstances the servo motor may over-travel and cause further logged faults. This may be cured, following fault rectification, by cycling the ignition ON-OFF-ON 3 times Refer to PDU
33 Cool air by-pass potentiometer fault Harness / connector fault  In certain circumstances the servo motor may over-travel and cause further logged faults. This may be cured, following fault rectification, by cycling the ignition ON-OFF-ON 3 times Refer to PDU
34 Defrost vent potentiometer fault Harness / connector fault  In certain circumstances the servo motor may over-travel and cause further logged faults. This may be cured, following fault rectification, by cycling the ignition ON-OFF-ON 3 times Refer to PDU
35 Centre vent potentiometer fault Harness / connector fault  In certain circumstances the servo motor may over-travel and cause further logged faults. This may be cured, following fault rectification, by cycling the ignition ON-OFF-ON 3 times Refer to PDU
36 Foot vent potentiometer fault Harness / connector fault  In certain circumstances the servo motor may over-travel and cause further logged faults. This may be cured, following fault rectification, by cycling the ignition ON-OFF-ON 3 times Refer to PDU
41 LH fresh / recirc. motor fault Harness / connector fault Servo motor seized or sticking Flap seized or sticking Refer to PDU
42 RH fresh / recirc. motor fault Harness / connector fault Servo motor seized or sticking Flap seized or sticking Refer to PDU
43 Cool air by-pass motor fault Harness / connector fault Servo motor seized or sticking Flap seized or sticking Refer to PDU
43 Cool Air by-pass motor fault Harness / connector fault Servo motor seized or sticking Flap seized or sticking Refer to PDU
44 Defrost vent motor fault Harness / connector fault Servo motor seized or sticking Flap seized or sticking Refer to PDU
45 Centre vent motor fault Harness / connector fault Servo motor seized or sticking Flap seized or sticking Refer to PDU
46 Foot vent motor fault Harness / connector fault Servo motor seized or sticking Flap seized or sticking Refer to PDU
PDU DTC codes:  not generally accessible with a generic OBDII code reader but linked to the above may provide additional ‘Possible Causes’

[image: image1.emf]
[image: image2.emf]
[image: image3.emf]
[image: image4.emf]
